| 1.0 | OBJECTIVE : | To lay down the procedure for validation of Microsoft Excel Worksheets to be used for Analytical Calculations. |
| --- | --- | --- |
| 2.0 | ## RESPONSIBILITY : | QC Chemist / QC Head / QA Executive / QA Head |
| 3.0 | ### ACCOUNTABILITY : | QA Head |
| 4.0 | PROCEDURE : |  |
| 4.1 | Excel Spreadsheet Controls :- |  |
| 4.1.1 | Each Worksheet shall be Product and Test specific. |  |
| 4.1.2 | All the worksheets used in analytical calculations shall be validated by QC designee and Approved by QA department. |  |
| 4.1.3 | QC designee shall design and validate the worksheets for different tests and shall prepare “Validated Excel Sheet Index” as per Annexure-I. |  |
| 4.1.4 | Each validated spreadsheet used for analytical calculation shall be numbered as, IO-QC-XXXX-NN Where, - IO stands for Indiana Ophthalmics LLP, - QC stands for Quality Control, - XXXX stands for continuous serial number starting from 0001, - NN stands for version number starting with 00. |  |
| 4.1.5 | Master Excel Spreadsheet shall be stored in a separate folder on the computer of QC designee. |  |
| 4.1.6 | Only QC designee shall have rights to create new spreadsheet, modification and deletion in master worksheets. |  |
| 4.1.7 | Master Excel worksheet shall be used for the purpose of calculations by the QC Analysts from the computer terminals in the laboratory. |  |
| 4.1.8 | Entry shall be allowed in only those cells which are unprotected. |  |
| 4.1.9 | Unprotected cells shall be highlighted with grey colour which is intended to be filled during calculation for easy identification. |  |
| 4.2 | Spreadsheet Protection :- |  |
| 4.2.1 | Each Master Spreadsheet shall be password protected. |  |
| 4.2.2 | The password shall be consisting of six characters having alphanumerical in nature and starting with capital alphabet. |  |
| 4.2.3 | The password of each spreadsheet shall be maintained as per Annexure-II (Spreadsheet Password Record). |  |
| 4.2.4 | Each Master spreadsheet shall be saved as “Read-only” format. |  |
| 4.3 | Validation Procedure :- |  |
| 4.3.1 | Availability of Folder :- |  |
| 4.3.1.1 | Excel sheet shall be available on a separate folder. |  |
| 4.3.2 | Wrong Password :- |  |
| 4.3.2.1 | An attempt shall be made to open Excel Spreadsheet using a wrong password. |  |
| 4.3.2.2 | Acceptance Criteria: The Excel Spreadsheet shall not open with the wrong password and shall display the message accordingly. |  |
| 4.3.3 | Alteration :- |  |
| 4.3.3.1 | An attempt shall be made to alter the contents of the protected cells, carrying the formula and fixed contents. |  |
| 4.3.3.2 | Acceptance Criteria: The Excel worksheet shall not allow alteration of contents in the protected cells and the system shall display a message accordingly. |  |
| 4.3.4 | Addition or Deletion of row/column :- |  |
| 4.3.4.1 | An attempt shall be made to add or delete row/column. |  |
| 4.3.4.2 | Acceptance Criteria: The Excel worksheet shall not allow adding or deleting the row/column and shall display the message accordingly. |  |
| 4.3.5 | Verification :- |  |
| 4.3.5.1 | Each of the calculations shall be done by calculator as well as excel sheet and result shall be compared. |  |
| 4.3.5.2 | Results shall be rounded-off up to pre-determined decimal places. |  |
| 4.3.5.3 | Calculations done by the calculator shall be recorded. |  |
| 4.3.5.4 | Acceptance Criteria: There shall be no difference in calculated values using Excel Worksheet and Calculator. |  |
| 4.3.6 | Validation activity shall be recorded in “Validation of Excel Spreadsheet” as per Annexure-III. |  |
| 4.4 | Revalidation Criteria :- |  |
| 4.4.1 | Excel Sheets shall be re-validated at every 5 years±30 days. |  |
| 4.4.2 | Excel sheets shall be revalidate in following cases:- |  |
|  | - Any change in the formula which will impact the result. |  |
| 4.4.3 | Certain changes like text matter / addition of unit / other descriptive information (not limited to) to excel sheet may not necessitate additional or re-validation. |  |
| 5.0 | ANNEXURES:- |  |
| 5.1 | Annexure-I : Validated Excel Sheet Index |  |
| 5.2 | Annexure-II : Spreadsheet Password Record |  |
| 5.3 5.4 | Annexure-III : Validation of Excel Spreadsheet Annexure-IV : Periodic Re-verification of Excel Sheet Validation |  |
| 6.0 | REFERENCES:- NA |  |
| 7.0 | REVISION HISTORY:- |  |
| Sr. No.; Revision No.; Date of Revision; SOP No.; Supersedes No.; Reason for Revision. 02; 09/11/2022; QAGE116-02; QAGE116-1; - Change in Firm Name from “Indiana Ophthalmics” to “Indiana Ophthalmics LLP”. - Addition of Annexures, References and Revision History as Section 5.0, 6.0 and 7.0 respectively in the SOP.. |