| 1.0 | OBJECTIVE : | This procedure describes the system for carrying out validation of cleaning procedures of manufacturing equipment which will assess the cleanliness of the equipment used in all the critical manufacturing steps where the product is being manufactured and to ensure that the equipment is consistently cleaned to an acceptable level of product and microbial residue. |
| --- | --- | --- |
| 2.0 | SCOPE : | This procedure is applicable for cleaning validation study to be performed in Indiana Ophthalmics LLP. |
| 3.0 | ## RESPONSIBILITY : | All departmental head to ensure the compliance of this SOP, Officer / Executive QA |
| 4.0 | ACCOUNTABILITY : | QA Head |
| 5.0 | PROCEDURE : |  |
| 5.1 | Definition :- |  |
| 5.1.1 | Cleaning Validation:- Cleaning validation is a documented process that verifies the effectiveness of a cleaning procedure to remove residues, contaminants, and impurities from equipment, surfaces, and systems to a predetermined level of cleanliness. |  |
| 5.1.2 | Cleaning Verification:- A process for determining the effectiveness of a cleaning process for a specific cleaning event. It is generally a single run study. |  |
| 5.1.3 | Equipment Train:- Equipment train is defined as “Group of equipment in sequence, utilized for manufacturing particular group of product, considering the batch size and capacity of equipment”. The sequence of equipment through which a product is produced or processed. |  |
| 5.1.4 | Worst Case:- The product selected from a group of product that represents a greater risk of carry over contamination to other product utilizing same equipment by virtue of its solubility, toxicity, potency or a combination of these factors. |  |
| 5.1.5 | Rinse Sampling:- Procedure for sampling involving flooding the surface with rinse solution to effectively remove target residue. The rinse solution is then analyzed for target residue. |  |
| 5.1.6 | Swab Sampling:- Procedure for sampling surface involving wiping the surfaces with swab, typically saturated with water or another sampling solvent, to remove residues from a surface. The swab is then desorbed, and chemical analysis is performed on the desorbed material. |  |
| 5.1.7 | Clean :- Having product residue, process residue and environmental contaminants removed to an acceptable level. |  |
| 5.1.8 | Recovery Study :- A laboratory study combining the sampling method and analytical method to determine the quantitative recovery of a specific residue for a defined surface. |  |
| 5.1.9 | Residue :- Chemical or Microbiological material remaining on equipment surface after a cleaning process. |  |
| 5.1.10 | Acceptance Criteria :- To demonstrate during validation that the cleaning procedure, routinely employed for a piece of equipment, limits potential carryover to an acceptable level. |  |
| 5.1.11 | LD50 :- The 50% lethal dose of the target residue in an animal, typically in mg/kg of body weight (by the appropriate route of administration). |  |
| 5.1.12 | NOEL :- No observed effect level is the highest tested dose at which no “critical” effect is observed. |  |
| 5.1.13 | Permitted Daily Exposure (PDE):- The PDE is defined as the maximum amount of a substance that can be safely inhaled or ingested by a worker over a 24-hour period, without causing adverse health effects |  |
| 5.1.14 | Clean Equipment Hold Time (CEHT):- The maximum allowable holding time of equipment post cleaning and before manufacturing. |  |
| 5.1.15 | Dirty Equipment Hold Time (DEHT) :- The maximum allowable hold time of soiled equipment after completion of the manufacturing process prior to cleaning. |  |
| 5.1.16 | Critical Process Parameter:- A process parameter whose variability impacts a quality attribute and therefore needs to be controlled to ensure the process produces the desired quality output. A critical process parameter remains critical even if it is controlled. For cleaning validation, examples include chemistry/concentration of cleaning agent, temperature, time, action (e.g., pressure/flow rates) |  |
| 5.1.17 | Cleanability:- A factor representing the relative difficulty of removing a product formulation, inclusive of all active and inactive ingredients, from equipment surfaces utilizing specified cleaning procedures. |  |
| 5.1.18 | Health Based Exposure Limit:- Maximum Allowable Carryover Limit considering health based exposure i.e.PDE value |  |
| 5.1.19 | Lowest Therapeutic Dose (LTD) :- The lowest dose required to produce the desired effect. |  |
| 5.1.20 | Maximum Safe Carryover (MSC) /Threshold Limit Value (TLV) :- Maximum amount of residue that may be present in subsequent product. MSC is based on PDE of analyte being cleaned. A Mathematical calculation is performed for establishing acceptance limits that allows a certain fraction of the therapeutic dose to carry over into the maximum daily dose of the following product. |  |
| 5.1.21 | Maximum Daily Dose:-For the purpose of this SOP, MDD of a product is maximum dose of complete formulation of a strength that can be taken in a day. It is determined by considering unit weight of product (API + Excipients) multiplied by maximum number of units of the strength of product that can be taken per day |  |
| 5.1.22 | Periodic Monitoring :- A periodic single-cycle cleaning monitoring study completed on worst case product based from respective equipment group, where possible, to confirm that cleaning remains efficacious |  |
| 5.1.23 | Product Contact Parts:- All parts of manufacturing/packaging equipment that directly contact materials/product during manufacturing. |  |
| 5.1.24 | Solubility:- The degree to which a substance dissolves into a known quantity of solvent. For the purposes of this SOP, solubility shall be taken preferably from PharMSCpoeia, if available or from authentic reference like Merck index or Martindale Database or provided by R&D or in-house document. |  |
| 5.1.25 | Toxicity :- The type and extent to which a given amount of a substance can damage an organism |  |
| 5.1.26 | Worst Case Equipment:- Equipment having similar design and function and same cleaning procedure are grouped together. In a group a representative equipment which is having largest capacity or product contact area is a worst-case equipment for that group. Once cleaning validation performed on worst case equipment, same shall be applicable to the group of equipment represented by the worst-case equipment |  |
| 5.1.27 | Dose Criteria :- Not more than 0.001 smallest/lowest dose of any product shall appear in the maximum daily dose of next product being manufactured |  |
| 5.1.28 | 10 ppm Criteria:- Not more than 10 ppm of any product shall appear in next product being manufactured. |  |
| 5.2 | CLEANING VALIDATION PROCEDURE :- |  |
| 5.2.1 | Validation in consultation with the Head QA, Head QC and Head Production shall initiate the exercise of validation of the cleaning procedure for the specific equipment or equipment chain. |  |
| 5.2.2 | The cleaning validation exercise shall be carried out on the basis of a specific written authorized validation protocol that sets out the details of activities to be carried out during the validation. |  |
| 5.3 | CLEANING VALIDATION OF WORST CASE PRODUCT :- |  |
| 5.3.1 | Prerequisites |  |
| 5.3.1.1 | All the process equipment, cleaning equipment and utilities must be qualified. |  |
| 5.3.1.2 | Equipment design must be ensured during equipment qualification that the equipment is cleanable. |  |
| 5.3.1.3 | Information on equipment design, materials of construction (MOC’s) and their associated surface areas must be available for all surfaces being cleaned, and these are documented as a part of equipment qualification. |  |
| 5.3.1.4 | Approved and implemented cleaning procedures for equipment and accessories shall be in place. Any new design equipment which has not been experienced previously, cleaning process shall be defined with the help of recommendations from equipment manufactures or through engineering runs during equipment qualification and same shall be documented. |  |
| 5.3.1.5 | Product related information shall be available that includes batch size, dose information, process equipment train, API solubility in Water, Potency, PDE value. |  |
| 5.3.1.6 | PDE, lowest therapeutic dose and maximum daily dose value for all the API shall be available at site. |  |
| 5.3.1.7 | Validated analytical method for residue testing shall be available. |  |
| 5.3.1.8 | All personnel involved in cleaning of equipment and in cleaning validation shall be trained in respective procedures. |  |
| 5.3.2 | Cleaning Validation for new product |  |
| 5.3.2.1 | PDE, lowest therapeutic dose and maximum therapeutic dose value for each new molecule planned to introduce at site shall be made available at the site. |  |
| 5.3.2.2 | On introduction of a new product at site, an assessment to determine risk ranking Annexure II “Product Information Sheet-New Product Introduction” shall be done. The assessment determines whether the molecule is having highest risk ranking score among all the products manufactured at site. |  |
| 5.3.2.3 | If assessment performed identifies that the product is worst case for any equipment / group, cleaning validation shall be performed on the new product during commercial scale manufacturing with three consecutive cleaning cycles on the worst-case equipment. |  |
| 5.3.2.4 | Cleaning Validation Matrix shall be updated with revision in version number to establish acceptance criteria based on PDE value for cleaning validation .The same shall be included or referenced in protocol |  |
| 5.3.2.5 | Cleaning validation protocol shall be prepared as per Annexure No. X ‘Cleaning Validation Protocol’ |  |
| 5.3.2.6 | Cleaning shall be performed by production department as per the respective equipment cleaning SOP. All the variable process parameters (for e.g. Temperature and Time) shall be monitored. |  |
| 5.3.2.7 | After completion of cleaning activity, production department shall ensure equipment is visually clean and dry. Swab sample request shall be prepared by the Production as per SOP No. QAGE50 and submit to IPQA. |  |
| 5.3.2.8 | IPQA personnel shall verify that the equipment is visually clean and dry as per site specific procedure for line clearance. IPQA shall certify the equipment for visual clearance |  |
| 5.3.2.9 | Failure of a visual inspection during cleaning validation/verification/periodic monitoring shall be investigated as per SOP for Incident handling procedure, SOP No. QAGE118. |  |
| 5.3.2.10 | Trained personnel for microbial swab sampling shall collect microbial swab sample as per SOP No. QAMI23 “Bioburden of Equipment” from the identified locations before chemical residue swab sampling |  |
| 5.3.2.11 | IPQA shall collect the swab sample for chemical residue from the identified sampling location as per SOP No. QAGE50 for ‘Swab sampling. |  |
| 5.3.2.12 | The location where micro and chemical swab sampling is required to be performed, chemical swab sampling shall be done adjacent to microbial sampling location. |  |
| 5.3.2.13 | Quality Control shall perform analysis for the previous product API residue, microbial limits and cleaning agent (if used) following methods as per STP. |  |
| 5.3.2.14 | QC shall prepare analytical report for residue results for chemical and microbial testing. |  |
| 5.3.2.15 | If the results are outside the acceptance criteria, investigation shall be performed as per SOP No. QAGE113 on “Handling of Laboratory Incident” to identify the root cause and further disposition action shall be taken based on outcome of investigation. |  |
| 5.3.2.16 | Cleaning validation report shall be prepared as per Annexure XI ‘Cleaning Validation Report’ by compiling the analytical and microbial results for a product after completion of three runs. |  |
| 5.3.2.17 | If the new product is not a worst-case product in any criteria for any equipment group, cleaning validation study is not required however cleaning validation matrix shall be updated to include the new molecule as next product to evaluate the impact of new molecule on current acceptance criteria for the equipment group. |  |
| 5.3.2.18 | If the determined acceptance criteria for existing worst case product is changed to stringent level due to introduction of new product, an assessment shall be performed on existing cleaning validation and analytical method validation data for compliance to the changed limit. Based on the outcome, further course of action shall be decided that may include change in cleaning procedure, assessment of suitability of analytical method for residue testing and revalidation of cleaning process, if the existing data are not in compliance to the changed limit. |  |
| 5.3.3 | Equipment Grouping |  |
| 5.3.3.1 | Equipment which can be clearly justified and documented as similar in design, function and same cleaning procedure shall be grouped together for cleaning validation studies. For example: If more than one Manufacturing tank are available at site with similar design (varied sizes/contact area), functionality and same cleaning procedure those can be grouped together. |  |
| 5.3.3.2 | The equipment grouping document shall be prepared as per Annexure IV “Grouping of Equipment’s for Cleaning Validation”. |  |
| 5.3.3.3 | Assessment shall be performed to identify worst case equipment for each group. Worst case equipment in the group shall be the one with maximum product contact surface area. Any other criteria, if used, shall be justified in grouping document. |  |
| 5.3.3.4 | Product contact part surface area and MOC shall be taken from equipment qualification document sourced from vendor or determined in-house and same shall be compiled in Annexure No. VI “Equipment Contact Surface Area and Total Contact Surface Area for Worst Case Product”. |  |
| 5.3.3.5 | Full cleaning validation (i.e. 3 consecutive successful runs) shall be performed on each worst-case equipment of each equipment group. |  |
| 5.3.3.6 | If a group is having multiple equipment exactly matching with worst case equipment (same design, function, cleaning procedure and product contact surface area), execution of cleaning validation on any or in parts on those equipment can be considered to complete three runs of the product changeover. |  |
| 5.3.3.7 | Equipment Grouping document shall be updated to incorporate the new equipment either in the existing group or by creating new group, based on evaluation as per Annexure IV “Grouping of Equipment for Cleaning Validation”. |  |
| 5.3.4 | Part A :- Determination of worst case product :- |  |
| 5.3.4.1 | Products manufactured and packaged at site that utilizes the same cleaning procedures are assessed for their risk of contaminating the subsequent different product based on the toxicity (PDE Banding), solubility of API in water and potency of API |  |
| 5.3.4.2 | Worst case product for cleaning validation is the product containing worst case API which is identified through the procedure delineated in subsequent points |  |
| 5.3.4.3 | Worst case product matrix shall be prepared for a group of products manufactured in a particular area using specific set of equipment. Products are grouped according to category of product and dosage form. These product groupings are further subdivided by types of equipment used in their manufacture. |  |
| 5.3.4.4 | Products shall be grouped according to dosage form. |  |
| 5.3.4.5 | Once the product groups have been established, the next step shall be to determine the “Worst-case” representative of each group, at this stage the worst-case determination is done based on the characteristic of drug and is independent of equipment chains. The criteria applied for the selection of the worst-case are- a) Permitted Daily Exposure (PDE) of Active Pharmaceutical Ingredient (API). b) Solubility data of the API used in drug product. b) Minimum Therapeutic dose of the drug (Potency). Note :- - If a group consisted of low dose active & high dose active then the low dose would be the worst-case. - If a group composed of 5 products of similar potency, the worst-case selection shall be done on the basis of PDE value. If PDE value is also same, product with least soluble API shall be considered as worst case. If PDE value and solubility of the identified molecules is same, consider all identified APIs for cleaning validation. - If the group composed of similar API with different strength, then the worst-case product shall be the highest concentration (API) of that product. |  |
| 5.3.4.6 | For determination of worst case product, toxicity (PDE Banding), solubility and potency of API shall be evaluated as detailed below. |  |
| 5.3.4.7 | Toxicity of API represented by PDE Banding that is derived from PDE value of API. |  |
| 5.3.4.8 | Solubility of API in water (As per Pharmacopoeia / Martindale database/Merck Index/In-house). |  |
| 5.3.4.9 | Potency (Lowest Therapeutic Dose of API ). |  |
| 5.3.4.10 | Each product (API wise in case of multiple API(s) in a product) shall be rated numerically within a range of 1-5 for each of the following risk factors. |  |
| 5.3.4.11 | Toxicity (PDE Banding): Rating for evaluation of PDE shall be done as defined in Table 1, where 1 is least toxic and 5 is most toxic. Table 2 PDE Banding; Ratings. More than 10 mg/day; 1. From 1 mg/day to 10 mg/day; 2. From 100 µg/day to <1 mg/day; 3. From 10 µg /day to <100 µg /day; 4. Less than 1 µg /day to <10 µg /day; 5. |
|  | Solubility: API solubility rating shall be done as defined in Table 2 Solubility Rating; Descriptive Terms; Approx. Quantities of solvent by volume of 1 Part of solute by weight. 1; Very Soluble; Less than 1 part. 2; Freely Soluble; From 1 to 10 Parts. 3; Soluble; From 10 to 30 parts. 4; Sparingly Soluble Slightly Soluble; From 30 to 100 parts from 100 to 1000 parts. 5; Very Slightly Soluble Practically Insoluble; From 1000 to 10 000 parts more than 10 000 parts. |
|  | Potency (Lowest Therapeutic Dose): Rating for evaluation of Potency shall be done as defined in Table 3 Table 3 Potency Rating; Included Dose Intervals (LTD_ Lowest Therapeutic Dose). 1; ≥ 100 mg. 2; ≥ 20 mg to <100mg. 3; ≥ 5mg to < 20mg. 4; ≥ 1mg to < 5mg. 5; < 1mg. |
| 5.3.4.12 | Each risk factor is weighted utilizing a numerical multiplier based on its impact. Toxicity has highest impact on cleaning validation, hence is weighted with 6, next is solubility weighted with 5 and Potency weighted with 4. |  |
| 5.3.4.13 | Add all three weighted ratings to yield one weighted total risk score for each molecule (API). |  |
| 5.3.4.14 | Worst case product is the product containing API with the largest weighted total risk score. |  |
| 5.3.4.15 | In case of multiple API product, if any one API appears as worst case, the product shall be considered as worst-case product. |  |
| 5.3.4.16 | If two or more products (API) share the largest risk score, product with least PDE value (more toxic) shall be considered as worst case. If PDE value is also same, product with least soluble API shall be considered as worst case. If PDE value and solubility of the identified molecules is same, consider all identified APIs for cleaning validation. |  |
| 5.3.5 | Part B :- Equipment Chain used for the manufacturing of a common group of Products :- |  |
| 5.3.5.1 | Worst case product(s) for each equipment group shall be identified individually based on - Product with Highest Risk Ranking Score - Product with molecule having the Least PDE value - Product with molecule having Least Solubility |  |
| 5.3.5.2 | All the products processed at sites shall be included in Process equipment train as per format Annexure No. VII to evaluate the product for identification of worst case molecule based on criteria mentioned in 5.3.3 for each equipment group |  |
| 5.3.5.3 | Cleaning validation shall be performed for each equipment group on identified worst case product as derived through procedure mentioned in 5.3.4. |  |
| 5.3.5.4 | If identified worst case product is not in plan or having less number of batches to complete cleaning validation or If no manufacturing plan for worst case product- Next worst case product which is routinely manufactured having highest risk score among all other products going in that equipment group shall be identified to perform the cleaning validation. |  |
| 5.3.5.5 | Cleaning validation shall be completed for worst case product as and when manufactured |  |
| 5.3.5.6 | If worst case product is in plan but having less number of batches- Cleaning validation activity shall be initiated on available number of batches and interim report shall be prepared. Cleaning validation shall be completed as and when product is manufactured. |  |
| 5.3.5.7 | Additionally, next worst case product shall be identified which is routinely manufactured and cleaning validation shall be performed. |  |
| 5.3.5.8 | For dedicated equipment, tools & utensils, cleaning validation is not required. |  |
| 5.3.6 | Part C :- Determination of Maximum Safe Carryover and Acceptance criteria :- |  |
| 5.3.6.1 | Maximum Safe Carryover (MSC) shall be calculated using following criteria:- - Toxicity Criteria (based on PDE) - Dose Criteria - PPM Criteria - Visual Clean Criteria |  |
|  | The Permitted Daily Exposure (PDE) can be calculated based on the data and information obtained. For example |  |
|  | PDE = NOEL X WEIGHT ADJUSTMENT F1 x F2 x F3 x F4 x F5 Where NOAEL is no-observed adverse effect level, and F represents various adjustment factors. The value selected for each factor should be justified. All adjustment factors should ideally be compound-specific. Default values should only be used where no compound-specific data are available. - The PDE is derived by dividing the NOAEL for the critical effect by various adjustment factors (also referred to as safety-, uncertainty-, and assessment- or modifying factors) to account for various uncertainties and to allow extrapolation to a reliable and robust no-effect level in the human or target animal population. (Note: The values for the factors cited below are defaults and should only be used in the absence of compound-specific information). F1 to F5 are addressing the following sources of uncertainty: - F1: A factor (values between 2 and 12) to account for extrapolation between species; - F2: A factor of 10 to account for variability between individuals; - F3: A factor 10 to account for repeat-dose toxicity studies of short duration, i.e., less than 4-weeks; - F4: A factor (1-10) that may be applied in cases of severe toxicity, e.g. non-genotoxic carcinogenicity, neurotoxicity or teratogenicity; - F5: A variable factor that may be applied if the no-effect level was not established. When only an LOEL is available, a factor of up to 10 could be used depending on the severity of the toxicity |  |
| 5.3.6.2 | Residue acceptance limit based on PDE for worst case product is determined by using maximum allowable carryover formulae (MSC) |  |
|  | PDEA (mg) X Batch sizeB (kg) X 106 (conversion factor kg to mg) MSC in mg = Maximum daily dose of next productB (mg). |
|  | Where, “A” : Product being cleaned (worst case API) “B” : Product manufactured after product being cleaned. PDEA : Permitted Daily Exposure of product being cleaned (PDE value of identified worst case product in mg/day BatchB : Batch size of product B (kg) DoseB : Maximum daily dosage strength product B including full dose weight with all excipients (mg) LTD : Lowest/smallest Therapeutic Dose of Previous product (mg) |  |
| 5.3.6.4 | After calculating the MSC value, Maximum Safe Surface Residue (MSSR) (µg/swab) shall be calculated using the following formula : Step-2 Maximum Safe Surface Residue (MSSR) (µg/swab) = - MSC (mg) X 25 cm2 (swab area) X 1000 (Conversion factor from mg to µg) TSA (cm2). Where, “MSC” : Maximum Safe carryover calculated in step-1 Swab sampling area (cm2): Shall be 25 cm2 1000 : Conversion factor from mg to µg TSA = Total surface area of equipment in cm2 |
| 5.3.6.5 | Dose Criteria :- As per this criteria NMT 1/10000 (Safety factor) part of lowest therapeutic dose of previous product should not contaminate maximum daily dose of next product. - The maximum allowable carry over (MSC) shall be expressed as: MSC= STD X SF X SBS LDD Where MSC = Maximum Safe carryover of previous product in to next product in mg. STD = Smallest therapeutic dose of previous product in mg. SF = Safety factor (1/10000). SBS = Smallest batch size of any product manufacturing in equipment train (Product-B). LDD = Largest daily dose of any product manufacturing in equipment train (Product-B). To make Acceptance Criteria more stringent total surface area of all equipments to be considered during calculation. - Maximum Safe Surface Residue (MSSR) per swab shall be expressed as: MSSR = MSC X SWAB AREA TSA Where Swab area = Area of equipment selected for one swab sample in cm2 (25 cm2). TSA = Total surface area of equipment in cm2. |  |
| 5.3.6.7 | Limit by swab sampling i.e. Total Viable Aerobic Count (Total of bacterial count and Yeast and mold Count) shall as per below table. Grade; TAMC Limit (CFU/Swab); TYMC Limit (CFU/Swab). A; <1; NIL. B; 5; NIL. C; 25; NIL. D; 50; NIL. |
| 5.3.6.8 | PPM Criteria: - Not more than 10 PPM of any product residue shall appear in another product manufactured subsequently. - The maximum safe carry over (MSC) per swab shall be expressed as: MSC = 10 X SBS X SWAB AREA TSA Where, Swab area = Area of equipment selected for one swab sample in cm2 (25 cm2). SBS = Smallest batch size of any product manufacturing in equipment train (Product-B). TSA = Total surface area of equipment in cm2. |  |
| 5.3.6.9 | By Visual Inspection: - No quantity of residue will be visible on equipment after cleaning procedure is performed. |  |
| 5.3.6.10 | Calculating Maximum Safe Surface Residue (MSSR) in Rinse Solution |  |
|  | MSC (mg) X 1000 MSSR = Total Rinse Solution. |
| 5.3.7 | Acceptance criteria :- |  |
| 5.3.7.1 | After determining the MSC values, the (most stringent value of MSC) lowest MSC value obtained from Toxicity criteria, Dose Criteria, 10 PPM Criteria, and Visual Criteria shall be selected as acceptance criteria. |  |
| 5.3.7.2 | If the MSC Value arrived as explained is below Limit of Quantitation then acceptance Criteria for validation study shall be the Limit of Quantitation. |  |
| 5.3.7.3 | Direct surface swabbing and / or rinse sampling methods shall be used for cleaning validation studies. Direct surface sampling shall be preferred for sampling the areas hardest to clean and which are reasonably accessible. Rinse sampling shall be used for large surface areas and for inaccessible systems, which are not routinely dismantled. |  |
| 5.3.7.4 | The cleaning validation shall be performed on three cycles / runs for the selected worst-case product. |  |
| 5.3.7.5 | For cleaning validation both chemical (swab and rinse sample) and microbiological (swab sample) analysis shall be performed. |  |
| 5.3.8 | Cleaning validation for non-dedicated accessories and tools |  |
| 5.3.8.1 | Sampling & dispensing tools, production accessories such as scoops, drums, sampling tools, gaskets and accessories which are not covered during cleaning validation, a protocol based study shall be carried out considering identified worst case product in the facility. |  |
| 5.3.8.2 | Cleaning shall be performed by the concerned personnel as per respective procedure. |  |
| 5.3.8.3 | Swab sampling (for Chemical residue and microbial load) request shall be raised by concerned department personnel and submitted to IPQA. |  |
| 5.3.8.4 | After visual clearance by IPQA personnel, same procedure as mentioned from 5.3.2.7 to 5.3.2.12 for the activity shall be followed. |  |
| 5.3.8.5 | Acceptance criteria shall be the lowest limit among the worst-case acceptance limit in the cleaning validation matrix and for microbial swab samples as per below table. Grade; TAMC Limit (CFU/Swab); TYMC Limit (CFU/Swab). A; <1; NIL. B; 5; NIL. C; 25; NIL. D; 50; NIL. |
| 5.3.8.6 | A cleaning validation report shall be prepared, reviewed & approved. |  |
| 5.3.8.7 | The study shall be repeated if a new product replaces existing worst case or change in MSC to stringent level and existing data do not comply to the changed MSC. |  |
| 5.3.9 | CHOICE OF CLEANING AGENT USED |  |
|  | Purified (Distilled) water is used as cleaning agent for cleaning of equipment’s due to below mentioned characteristics |  |
| 5.3.9.1 | Solubility of material to be removed: Solubility of Material to be removed shall be determined from below mentioned table: Descriptive Terms; Approx. Quantities of solvent by volume of 1 Part of solute by weight. Very Soluble; Less than 1 part. Freely Soluble; From 1 to 10 Parts. Soluble; From 10 to 30 parts. Sparingly Soluble Slightly Soluble; From 30 to 100 parts from 100 to 1000 parts. Very Slightly Soluble Practically Insoluble; From 1000 to 10 000 parts more than 10 000 parts. |
| 5.3.9.2 | Design & Construction of the equipment and surface materials to be cleaned |  |
| 5.3.9.2.1 | All the equipment’s involve in the cleaning validation study having smooth finish and easy to clean surface |  |
| 5.3.9.2.2 | All the equipment MOC should be SS316L and SS304 which is non-reactive to any chemicals or material |  |
| 5.3.9.2.3 | Gasket should made up of Food Grade material and inert in nature. So, it does not react with cleaning agent |  |
| 5.3.9.2.4 | Equipment’s having very less dead space makes it easier for cleaning |  |
| 5.3.9.3 | Safety of using Purified Water (Distilled Water) as cleaning agent: |  |
| 5.3.9.3.1 | It is safe to use Purified Water as cleaning agent as Purified (Distilled) Water complies with all the different pharmacopeia requirement like IP/BP/IH with respect to Chemical and Microbiological Tests .Chemical test includes - Description, - Acidity or Alkanity, - Ammonium, Calcium & Magnesium, - Heavy Metals, - Chloride, Nitrate, Sulfates, - Ox disable Substance, - Residue on evaporation, - pH, - Conductivity - Microbial Contamination test includes Total Microbial Count & Total Mould & Yeast Count and Pathogens |  |
| 5.3.9.3.2 | These tests are carried out on daily basis. Hence, we are using Purified (Distilled) Water as cleaning agent |  |
| 5.3.9.3.3 | Environmentally friendly: Hot water is an environmentally friendly cleaning agent, as it is biodegradable and non-polluting |  |
| 5.3.9.4 | The Ease of removal & Detection |  |
| 5.3.9.4.1 | Ease of Removal - Solubility: Hot water is an excellent solvent, capable of dissolving and removing many contaminants. - Rinsing: Hot water can be easily rinsed off equipment surfaces, reducing the risk of residues |  |
| 5.3.4.9.2 | Detection - Visual inspection: Hot water is transparent, making it easy to visually inspect equipment surfaces for residual water spots or stains. - Conductivity testing: Hot water's conductivity can be measured to detect its presence on equipment surfaces. - Total Organic Carbon (TOC) analysis: TOC analysis can detect organic contaminants that may be present in hot water. - pH testing: pH testing can detect any changes in the pH of the equipment surface, indicating the presence of hot water |  |
| 4.3.9.5 | NUMBER OF CLEANING CYCLES TO BE PERFORMED |  |
| 4.3.9.5.1 | Regulatory Guidance - FDA Guidance: The FDA recommends a minimum of three consecutive cleaning validation batches to demonstrate the effectiveness of the cleaning process. - EU GMP Annex 15: The EU GMP Annex 15 recommends a minimum of three cleaning validation batches, but also suggests that more batches may be necessary for complex or critical equipment |  |
| 4.3.9.5.2 | Factors Influencing Batch Number - Equipment complexity: More complex equipment, such as equipment with multiple parts or hard-to-reach areas, may require more batches to ensure thorough cleaning. - Molecule properties: A low solubility and high potency, may require more batches to ensure effective removal. - Cleaning process variability: If the cleaning process has inherent variability, such as differences in cleaning agent concentration or temperature, more batches may be necessary to demonstrate consistency |  |
| 4.3.9.5.3 | Based on Risk Ranking Residual risk level; Description; Cleaning Process Knowledge; Cleaning Process Understanding; Control Strategy. Severe (5); Multiple factors have high risk ratings.; H; H; H. High (4); Few factors have high risk ratings or all have medium risk rating.; H; H; M. M; H; H. H; M; H. M; M; M. Moderate (3); Medium Risk Level for multiple factors or high risk level for one factor; H; M; M. M; H; M. M; M; H. M; M; L. L; M; M. M; L; M. Low (2); Medium risk level for a few factors, the others are low risk.; M; L; L. L; M; L. L; L; M. Minimal (1); Low risk level for all factors; L; L; L. |
| 4.3.9.5.4 | Residual risk level; Number of batches; Rationale. Severe (5); Not ready for Cleaning Validation; Additional development should be pursued to identify processes or controls needed to reduce residual risk.. High (4); 10; Higher residual risk makes it unlikely that a small number of Cleaning Validation batches are adequate to show process consistency. A large number of successful batches may show process consistency, but achieving this would be unlikely if controls are not adequate. A preferable course of action would be to perform additional development and/ or knowledge acquisition to reduce residual risk so that fewer Cleaning Validation batches would be needed.. Moderate (3); 5; Increased residual risk can be addressed by preparing two additional Cleaning Validation batches to provide further demonstration of process consistency.. Low (2); 3; Knowledge and control strategy are regarded as sufficient. Three Cleaning Validation batches have been shown historically to be appropriate for demonstrating cleaning process consistency .. Minimal (1); 1-2; Strong knowledge and high degree of controls minimize risk.. |
| 4.3.9.5.5 | The number of cleaning validation batches required for cleaning validation activity is determined based on a combination of regulatory guidance, equipment complexity, molecule properties, risk ranking and statistical considerations. A minimum of three consecutive batches is recommended to demonstrate effective cleaning. |  |
| 5.3.10 | Selection of Sampling Locations :- |  |
| 5.3.10.1 | The product contact part of the equipment, which is difficult to clean shall be selected for sampling as per Annexure IX title “Evaluation of Equipment”. |  |
| 5.3. 10.2 | The list of difficult to clean areas for various equipment’s shall describe in cleaning validation protocol. |  |
| 5.3. 10.3 | Sampling Locations for testing Microbial contamination on product contact parts of equipment. |  |
| 5.3. 10.4 | The list of sampling locations for various equipment’s shall describe in cleaning validation protocol. |  |
| 5.3.11 | Surface Area Calculation :- |  |
| 5.3.11.1 | The total surface area of product contact for each equipment’s shall be calculated as per Annexure VI “Equipment Contact Surface Area” with the help of engineers and actual measurement of various product contact parts of equipment. The same shall be used for MSC calculation. |  |
| 5.3.12 | Sampling Technique :- |  |
| 5.3.12.1 5.3.12.2 | - Swab preparation :- Use non sterile HI Clean Swab for residue sampling and sterile non-absorbent cotton swabs for microbial analysis. - Sample Area: 5 x 5 cm for Chemical residue & for Micro analysis sample. - Sampling patterns: Wipe the defined area in both the directions as shown in the figure. Apply only one time. Do not rub the surface in to & fro movement. Refer the typical diagram to collect the sample-using swab. (Direction of swabbing strokes) and (Fig.A) (Fig. B) |  |
| 5.3.12.3 | Sampling procedure for Residue for chemical analysis :- |  |
| 5.3.12.3.1 | During sampling, Chemist should wear hand gloves. |  |
| 5.3.12.3.2 | Take specified solvent in a test tube. |  |
| 5.3.12.3.3 | Dip non sterile HI Clean swab in specified solvent to wet/ moisten the swab and squeeze. |  |
| 5.3.12.3.4 | Swab critical areas of the specified equipment as per protocol. |  |
| 5.3.12.3.5 | Swab sampling location as defined in the protocol. |  |
| 5.3.12.3.6 | Swabs used for sampling should be dipped in the same solvent. |  |
| 5.3.12.3.7 | Stopper the test tube immediately. |  |
| 5.3.12.3.8 | Put identification label to the test tubes. |  |
| 5.3.12.4 | Sampling Procedure for Microbial Analysis :- |  |
| 5.3.12.4.1 | During sampling, Chemist should wear sterile hand gloves. |  |
| 5.3.12.4.2 | Take 5ml of buffered Sodium Chloride Peptone Solution in a sterile test tube. |  |
| 5.3.12.4.3 | Dip the swab in buffered Sodium Chloride Peptone Solution to wet/ moisten the swab. |  |
| 5.3.12.4.4 | Squeeze the swab slightly by pressing the swab on the inside wall of the test tube. |  |
| 5.3.12.4.5 | Swab critical areas of the specified equipment as per protocol. |  |
| 5.3.12.4.5 | Keep the swab sample after sampling in a sterile test tube containing buffered Sodium Chloride Peptone Solution . |  |
| 5.3.12.4.6 | Stopper the test tube. |  |
| 5.3.12.4.7 | Put identification number, location number and date of sampling. |  |
| 5.3.12.4.8 | Take the Stoppered test tubes to QC for Microbial testing. |  |
| 5.3.12.4.9 | Use the sampled swabs for further testing within one hour. |  |
| 5.3.13 | Analytical Method for Swab Analysis :- |  |
| 5.3.13.1 | Analytical Method for Chemical Analysis :- The analytical methods employed for detection and quantitation of residues shall be validated for following parameters. a) Precision, linearity, selectivity b) Limit of Quantitation c) Limit of Detection d) Recovery, by spiking with the analyte; and e) Reproducibility. f) Sample Stability |  |
| 5.3.13.2 | Recovery studies shall be performed for material representing 5% or greater of the product contact surface. Typical materials to be tested using material coupon may include, but are not limited to Stainless Steel, acrylic material, Rubber gasket and Silicon. Site must assess which materials are used and require recovery studies for each equipment train/process/product. |  |
| 5.3.13.3 | Recovery levels shall be not less than 75%. In the event recovery levels are less than 75%, changing the quantity of swab solvent or increasing the number of repeat swabbing at same site during cleaning method validation are options to consider, provided these are validated. |  |
| 5.3.13.4 | Before MSC calculation, LOD and LOQ of the worst case API shall be determined and should be less than calculated MSC value. Also, calculated residue limit from cleaning validation of a particular equipment shall be less than LOQ. |  |
| 5.3.14 | Analytical Method for Microbial Analysis :- |  |
| 5.3.14.1 | Cleaning procedure shall be considered to be validated when visual inspection, chemical and microbial analysis of swab samples complies with the acceptance limit. |  |
| 5.3.15 | Conclusion and Approval :- |  |
| 5.3.15.1 | Cleaning procedure shall be considered to be validated when visual inspection, chemical and microbial analysis of swab samples complies with the acceptance limit. |  |
| 5.3.15.2 | If the quantity of active in the swab for specific equipment is out of the acceptance criteria, cleaning procedure shall be repeated and even on repetition if the cleaning validation fails. Investigation shall be conducted, based on the investigation, SOP shall be revised or cleaning procedure or cleaning agent shall be modified if required. The revalidation shall be done with above modification. |  |
| 5.3.15.2 | Final summary report shall be approved prior to considering the cleaning process for routine use. |  |
| 5.3.16 | Periodic Monitoring :- |  |
| 5.3.16.1 | Periodic Monitoring shall consist of reverification of a minimum of one cycle of cleaning and sampling (chemical and microbial) for each equipment group at a minimum shall be performed Yearly (+/- 30 days), subjected to availability of production plan. Periodic verification shall be done on identified worst case product based on risk ranking in each equipment group. If highest risk ranking is not in manufacturing plan, other worst case identified in respective equipment group shall be considered. |  |
| 5.3.16.2 | Periodic monitoring shall be tracked using format for periodic monitoring tracker. Tracker shall be prepared at the beginning of year and shall be updated on inclusion of any new equipment/group or product. |  |
| 5.3.17 | Revalidation Criteria :- |  |
| 5.3.17.1 | The need for carrying out re-validation of cleaning procedure shall be assessed in the following situations by the quality assurance head in consultation with the validation team. |  |
| 5.3.17.2 | At the time of introduction of a new product: For this purpose a product having a new active not handled at the site earlier or same product with higher strength shall be considered as new product. |  |
| 5.3.17.3 | Change of equipment/modification in the existing equipment train. |  |
| 5.3.17.4 | Change in batch size of any of the product included in the matrix. |  |
| 5.3.17.5 | Change/modification in cleaning method/ cleaning agent and equipment. |  |
| 5.3.17.6 | Evidence of cleaning failure e.g. batch failure due to cross contamination etc. |  |
| 5.3.17.7 | Cleaning performance verification of the worst case products shall be carried out for each equipment chain once in every five years if the worst case is not changed. |  |
| 5.3.18 | Change of Equipment/modification in the existing equipment chain :- |  |
| 5.3.18.1 | Calculate surface area of new equipment. |  |
| 5.3.18.2 | Compare surface area with exiting value. |  |
| 5.3.18.3 | Prepared new equipment train & compare value of total surface area. |  |
| 5.3.18.4 | If revised value is less than earlier value than no need to change in cleaning validation. |  |
| 5.3.18.5 | If revised value is high than earlier value than need to calculate MSC again. |  |
| 5.3.18.6 | If the revised value is less than the earlier determined value, acceptance criteria shall be revised based on revised MSC value. |  |
| 5.3.18.7 | If the revised MSC value (acceptance criteria) is less than the observed highest MSC value obtained during the previous study then perform re-validation study using the current version of cleaning procedures. |  |
| 5.3.18.8 | If the above revalidation studies do not pass the revised MSC value (acceptance limit), the cleaning validation shall be repeated with the revised cleaning procedure following Change Control procedure. |  |
| 5.3.18.9 | If the revised MSC value found more than the earlier reported MSC value, then no cleaning validation shall be performed, as the earlier study proved that the cleaning procedure followed for each equipments stands effective and consistent to reduce previous product residue to acceptance level. |  |
| 5.3.19 | Change in Batch size of any of the product included in the matrix :- |  |
| 5.3.19.1 | Revised MSC matrix (Acceptance criteria) shall be prepared including the change in Batch size. |  |
| 5.3.19.2 | If the revised MSC value is less than the earlier determined MSC value, acceptance criteria shall be revised based on revised MSC value. |  |
| 5.3.19.3 | The revised MSC value shall be compared against MSC value reported in the earlier executed Cleaning Validation report. |  |
| 5.3.19.4 | If the revised MSC value found more than the earlier reported MSC value, then no cleaning validation shall be performed, as the earlier study prove that the cleaning procedure followed for each equipment stands effective and consistent to reduce previous product residue to acceptance level. |  |
| 5.3.19.5 | If the revised MSC value (acceptance criteria) is less than the observed highest MSC value obtained during the previous study then perform re-validation study using the current version of cleaning procedures. |  |
| 5.3.19.6 | If the above revalidation studies do not pass the revised MSC value (acceptance limit), the cleaning validation shall be repeated with the revised cleaning procedure following Change Control procedure. |  |
| 5.3.20 | Equipment Holding Period :- |  |
| 5.3.20.1 | The maximum acceptable holding times (with storage conditions) for the following shall be validated as part of the cleaning validation. |  |
| 5.3.20.2 | Dirty Equipment Hold Time (DEHT) :- |  |
| 5.3.20.2.1 | Hold time studies shall involve sampling for the presence of microbial growth during storage and after cleaning for product residue and for presence of microbial growth. |  |
| 5.3.20.2.2 | A protocol based study shall be performed for worst-case products to cover all the process trains. |  |
| 5.3.20.2.3 | The equipment shall be hold after batch change over cleaning on completion of a batch. |  |
| 5.3.20.2.4 | In case if some products are manufactured in long campaign and campaign length is established, DEHT shall also be performed after completion of the campaign by identifying worst case among the products manufactured in long campaign. |  |
| 5.3.20.2.5 | Swab sampling for microbial analysis shall be performed on identified location at zero time point and at different time interval as per approved protocol. |  |
| 5.3.20.2.6 | After cleaning of equipment again swab sampling shall be performed for microbial analysis and chemical residue analysis as per protocol. |  |
| 5.3.20.2.7 | The specifications for microbial growth and product residue shall be detailed in the protocol. |  |
| 5.3.20.2.8 | Results shall be compared against the limit specified in protocol for chemical and microbial analysis. |  |
| 5.3.20.2.9 | A report shall be prepared to recommend Dirty Equipment Hold time in number of days/hours. |  |
| 5.3.20.2.10 | In case worst case product is not manufactured in sufficient length, other product with proper justification shall consider and same shall be documented in protocol. |  |
| 5.3.20.2.11 | Activity shall also be done for accessories and tools used in dispensing, sampling considering worst case scenario, if not covered in manufacturing train. |  |
| 5.3.20.3 | Clean Equipment Hold Time (CEHT) :- |  |
| 5.3.20.3.1 | A protocol based hold time study shall be performed that involves sampling for the presence of microbial growth after product change over cleaning to establish the holding period for cleaned equipment. |  |
| 5.3.20.3.2 | Justification for sampling method(s) and limits shall be included in the validation protocol. |  |
| 5.3.20.3.3 | Clean Equipment Hold Time (CEHT) shall be established for the critical equipment (selected based on scientific rationale) stored for defined period. |  |
| 5.3.20.3.4 | Acceptance criteria shall be defined in the protocol with justification. |  |
| 5.3.20.3.5 | Swab sampling for microbial analysis shall be performed on identified locations at zero time point and at different time interval as mentioned in approved protocol for establishing the CEHT. |  |
| 5.3.20.3.6 | Result shall be compared against the limit defined in the protocol. |  |
| 5.3.20.3.7 | A report shall be prepared to recommend cleaned equipment hold time in hours/days. |  |
| 6.0 | ANNEXURES:- |  |
| 6.1 | Annexure-I : Review of PDE Report |  |
| 6.2 | Annexure-II : Product Information Sheet-New Product Introduction |  |
| 6.3 | Annexure-III : Impact Assessment on Equipment Groups |  |
| 6.4 | Annexure-IV : Grouping of Equipment for Cleaning Validation |  |
| 6.5 | Annexure-V : Products Risk Ranking |  |
| 6.6 | Annexure-VI : Equipment Product Contact Surface Area |  |
| 6.7 | Annexure-VII : Process Equipment Train |  |
| 6.8 | Annexure-VIII : Cleaning Validation Matrix |  |
| 6.9 | Annexure-IX: Evaluation of Equipment |  |
| 6.10 | Annexure-X: Protocol template for Cleaning Validation |  |
| 6.11 | Annexure-XI: Report template of Cleaning Validation |  |
| 6.12 | Annexure-XII: Protocol format for Periodic Monitoring |  |
| 6.13 | Annexure-XIII: Report format for Periodic monitoring |  |
| 6.14 | Annexure-XIV: Periodic Monitoring Tracker |  |
| 6.15 | Annexure-XV: Sampling Request Form |  |
| 7.0 | REFERENCES:- |  |
| 7.1 | WHO TRS 1019, 2019. |  |
| 7.2 | ISPE:- Risk-Based Manufacture of Pharmaceutical Products, IInd Edition September 2017. |  |
| 7.3 | EMA Guideline: Guideline on Setting Health Based Exposure Limits for use in Risk Identification in the Manufacture of Different Medicinal Products in Shared Facilities, 20 November 2014. |  |
| 7.4 | USFDA Guideline on Cleaning Validation: Guide to Inspections Validation of Cleaning Processes, FDA, 1993. |  |
| 7.5 | EU GMP guide Annex 15: Qualification and Validation. |  |
| 7.6 | PDA Technical Report No. 29: Points to Consider for Cleaning Validation Revised 2012 (TR 29). |  |
| 8.0 | REVISION HISTORY:- |  |
| Sr. No.; Revision No.; Date of Revision; SOP No.; Supersedes No.; Reason for Revision. 02; 31/03/2023; QAGE119-02; QAGE119-1; - Change in Firm Name from “Indiana Ophthalmics” to “Indiana Ophthalmics LLP”. - Addition of Scope, Annexures, References and Revision History as Section 2.0, 6.0, 7.0 and 8.0 respectively in the SOP.. 03; 02/01/2025; QAGE119-03; QAGE119-02; - Incorporation of PDE value for cleaning validation study. - Procedure updated as per WHO/EU guidelines. - Annexure-I to Annexure-XV newly incorporated. - Introduction of rating for evaluation of potency (Table 3) under the step 5.3.4.11. - Incorporation of cleaning validation for non-dedicated accessories and tools under the step 5.3.8.. |