| 1.0 | OBJECTIVE : | To describe step-wise procedure for the inspection of packed stock. |
| --- | --- | --- |
| 2.0 | ## RESPONSIBILITY : | IPQA Officer |
| 3.0 | ACCOUNTABILITY : | QA Head |
| 4.0 | PROCEDURE : |  |
| 4.1 | Packed stock inspection procedure includes the verification of the product quality defects related to the packaging of the product by ensuring the proper packing of the bottle / tube through random check of the total batch packed. |  |
| 4.2 | IPQA personnel shall perform packed stock inspection of packed product batch after completion of packaging activity of the respective batch. |  |
| 4.3 | Packed shippers shall be randomly selected as per n+1 sampling plan (Where n = total number of packed shippers). First and last shipper is mandatory for checking. |  |
| 4.4 | IPQA personnel shall enter date, product name, batch no., shipper no., shipper weight, minimum weight of shrink bag / outer box and maximum weight of shrink bag / outer box in “Packed Stock Inspection Register” as per Annexure-I. |  |
| 4.5 | All the shrink bags/inner boxes from selected shippers shall be checked for weight. Shrink bag/inner box shall be selected as per n+1 sampling plan (Where n = total number of shrink bag/inner box in the particular shipper) for quality parameter checking. Shrink bag/inner box with minimum and maximum weight from the particular shipper is mandatory for the verification. |  |
| 4.6 | Verification of following checkpoints shall be performed while packed stock inspection :- |  |
|  | - Integrity and Physical appearance of the corrugated box. |  |
|  | - Corrugated box label, legibility and correctness of coding details on it and on subsequent packing materials wherever coding is there. |  |
|  | - Integrity, physical appearance and legibility of batch details of the inner box/shrink pack. |  |
|  | - Correctness of quantity / number of units and sub-units in the corrugated box and subsequent inner packing. |  |
|  | - Check cutting and pasting of carton and inner box. |  |
|  | - Correctness of labeling and packing style. |  |
|  | - Leakage, any tear, breakage or damage of the product or of any packing material which warrants and exhibits quality not conforming to the set standards. |  |
|  | - Availability of leaflet in inner carton and legibility of leaflet. |  |
|  | - Proper adherence and correctness of batch details on the sticker label. |  |
|  | - Intactness of cap sealing. |  |
|  | - Check all the cartons, labels & inserts (if applicable) in pack for its correctness. |  |
|  | - Check overprinting on bottle-label, tube and carton (Batch No., Exp., Mfg. and M.R.P. (if applicable). |  |
|  | - Check leakage, nozzle deformation, presence of cap, absence of any foreign particles over the bottle/tube. |  |
|  | - Check crimping, colour peel-off, absence of dents, presence of cap and cap ring on tubes. |  |
| 4.7 | Repack all the cartons, shrinks/inner boxes and shippers after verification. |  |
| 4.8 | If no discrepancy observed during above checking, release the batch for BSR transfer. |  |
| 4.9 | In case of any discrepancy, check more number of shippers and the same shall be brought to the notice of QA Head and Packing supervisor and production head to initiate corrective and preventive actions. |  |
| 4.10 | IPQA shall record the packed stock inspection activity as per Annexure-I (Packed Stock Inspection Register). |  |
| 5.0 | ANNEXURES:- |  |
| 5.1 | Annexure-I : Packed Stock Inspection Register |  |
| 6.0 | REFERENCES:- NA |  |
| 7.0 | REVISION HISTORY:- |  |
| Sr. No.; Revision No.; Date of Revision; SOP No.; Supersedes No.; Reason for Revision. 02; 11/11/2022; QAGE120-02; QAGE120-1; - Change in Firm Name from “Indiana Ophthalmics” to “Indiana Ophthalmics LLP”. - Addition of Annexures, References and Revision History as Section 5.0, 6.0 and 7.0 respectively in the SOP.. |