| 1.0 | ### OBJECTIVE : | To lay down the procedure for Transport Validation. |
| --- | --- | --- |
| 2.0 | ### SCOPE : | This procedure is applicable for products manufactured at Indiana Ophthalmics LLP and dispatched to various locations. |
| 3.0 | ## RESPONSIBILITY : | Store Assistant, Store Officer, Store Head, QA Executive, IPQA Officer, QA Head, Dispatch in-charge |
| 4.0 | ACCOUNTABILITY : | QA Head |
| 5.0 | PROCEDURE : |  |
| 5.1 | One consignment of finished product (i.e., product whose storage condition is between 2 ⁰C - 8 ⁰C / NMT 25 ⁰C / below 30 ⁰C) shall be considered for the Transport Validation Study. |  |
| 5.2 | Temperature recording shall be carried out by using calibrated data loggers which are having sufficient memory for the intended duration of the study and the selected recording interval. |  |
| 5.3 | Program the wireless data logger through software. Set the time, date and recording interval in wireless data logger. |  |
| 5.4 | Data loggers shall be set in such a way that temperature shall be recorded at every 10 minutes interval during transportation. |  |
| 5.5 | The packaging of the consignment / product shall be carried out as per respective Batch Packaging Record. |  |
| 5.6 | Data loggers shall be placed inside the packed shipper by the dispatch personnel in the presence of QA personnel. Such shipper shall be marked with “X” (SHIPPER CONTAINING DATA LOGGER) to identify the shipper in which data logger is placed. |  |
| 5.7 | Shippers containing data loggers shall be placed in such a way that it covers corner area and center area during transportation. |  |
| 5.8 | The serial number of the data logger alongwith shipper number, in which it is placed, shall be noted for the traceability and further tracking. |  |
| 5.9 | The data logger shall record the temperature till the port of destination. |  |
| 5.10 | On receipt of the finished goods, by a representative of the party, the consignment shall be inspected for any visible damage from outside and shall record the observation if any. All the relevant entries shall be recorded as per Annexure-I (Finished Goods Receipt Checklist). |  |
| 5.11 | The representative of the party shall remove the data loggers from the shipper marked as “X” (SHIPPER CONTAINING DATA LOGGER). |  |
| 5.12 | Temperature readings shall be extracted from the data logger by programming through software. |  |
| 5.13 | Data loggers alongwith the extracted data shall be sent to the QA department of Indiana Ophthalmics LLP. |  |
| 5.14 | On receiving the data, QA department shall evaluate the minimum and maximum temperature attained during the transportation to establish the effect on the product. |  |
| 5.15 | A Transport Validation Protocol shall be prepared as per Annexure-II. It shall be numbered as below:- |  |
|  | TVP/PP/XY/NN |  |
|  | Where, |  |
|  | - TVP denotes Transport Validation Protocol, - PP denotes Product Name in 2 characters, - XY is either ED (in case of Eye Drop) or EO (in case of Eye Ointment), - NN is the version number starting from 00. |  |
| 5.15.1 | For example, Numbering of the first Transport Validation Protocol for the Eye Drop Product, Moxifloxacin Ophthalmic Solution, shall be assigned as TVP/MF/ED/00. |  |
| 5.16 | Acceptance Criteria :- |  |
| 5.16.1 | The shipper should maintain the integrity of the finished product. |  |
| 5.16.2 | The product consignment exposed to 2 ⁰C - 8 ⁰C / NMT 25 ⁰C / below 30 ⁰C throughout the transportation from manufacturing premises to the port of destination should maintain the quality of the finished product. |  |
| 5.16.3 | Temperature should be maintained at 2 ⁰C - 8 ⁰C / NMT 25 ⁰C / below 30 ⁰C throughout the transportation. The following criteria shall be considered in case of temperature excursion :- |  |
| 5.16.3.1 | For the product having storage condition of 2 ⁰C - 8 ⁰C, temperature excursion allows between 0 ⁰C to 10 ⁰C and the transient spike up to 12 ⁰C is permitted as long as they do not exceed 24 hours. |  |
| 5.16.3.2 | For the product having storage condition of NMT 25 ⁰C, temperature excursion allows between 15 ⁰C to 30 ⁰C and the transient spike up to 35 ⁰C is permitted as long as they do not exceed 24 hours. |  |
| 5.16.3.3 | For the product having storage condition of below 30 ⁰C, temperature excursion allows between 10 ⁰C to 35 ⁰C and the transient spike up to 40 ⁰C is permitted as long as they do not exceed 24 hours. |  |
| 5.16.4 | Any deficiency observed during the transport validation study shall be recorded in the report. Appropriate corrective and preventive actions shall be taken to prevent the recurrence of observed deficiency. |  |
| 5.16.5 | Results shall be documented based on the observations recorded during the transforming and evaluation of the results shall be carried out. |  |
| 5.16.6 | If the results meet the acceptance criteria established, it confirms that the quality of the finished product is maintained during transportation. |  |
| 5.16.7 | For temperature excursions, if the temperature shoots above the required 2 ⁰C - 8 ⁰C / NMT 25 ⁰C / below 30 ⁰C during the transit, the product sample shall be tested as per finished product specification of respective product. |  |
| 5.16.8 | The results should meet the release specifications. The deviation in the initial results and the results after excursion should not more than 1 %. |  |
| 5.16.9 | The product can be released for sale if the above criteria are compiled. |  |
| 5.17 | After completion of the study, Transport Validation Report shall be prepared which shall be numbered as below:- |  |
|  | TVR/PP/XY/NN |  |
|  | Where, |  |
|  | - TVR denotes Transport Validation Report, - PP denotes Product Name in 2 characters, - XY is either ED (in case of Eye Drop) or EO (in case of Eye Ointment), - NN is the version number starting from 00. |  |
| 5.17.1 | For example, Numbering of the first Transport Validation Report for the Eye Drop product, Moxifloxacin Ophthalmic Solution, shall be assigned as TVR/MF/ED/00. |  |
| 6.0 | ANNEXURES:- |  |
| 6.1 | Annexure-I : Finished Goods Receipt Checklist |  |
| 6.2 | Annexure-II : Transport Validation Protocol |  |
| 6.3 | Annexure-III : Transport Validation Report |  |
| 7.0 | REFERENCES:- |  |
| 7.1 | PIC/S PE 009-16 |  |
| 8.0 | REVISION HISTORY:- |  |
| Sr. No.; Revision No.; Date of Revision; SOP No.; Supersedes No.; Reason for Revision. 01; 31/12/2022; QAGE130-01; QAGE130-0; - Change in Firm Name from “Indiana Ophthalmics” to “Indiana Ophthalmics LLP”. - Addition of Scope, Annexures, References and Revision History as Section 2.0, 6.0, 7.0 and 8.0 respectively in the SOP.. |