| 1.0 | OBJECTIVE : | To lay down the procedure for the Continued Process Verification. |
| --- | --- | --- |
| 2.0 | SCOPE : | This procedure is applicable for continuous process verification of all the finished products manufactured at Indiana Ophthalmics LLP. |
| 3.0 | ## RESPONSIBILITY : | QA-Officer / QA-Executive |
| 4.0 | ### ACCOUNTABILITY : | QA Head |
| 5.0 | PROCEDURE : |  |
| 5.1 | Definition of terms : |  |
| 5.1.1 | Process Validation: Process Validation is defined as collection and evaluation of data, from the process design stage through commercial production, which establishes scientific evidence that a process is capable of consistently delivering quality products. |  |
| 5.1.2 | Continued Process Verification: Assuring that during routine production the process remains in a state of control. |  |
| 5.1.3 | Critical Process Parameters: A process input that, when varied beyond a limited range, has a direct and significant influence on a CQA, where a CQA is a quantitative property of an intermediate or final product that is considered critical for establishing the intended purity, efficacy and safety of the product. |  |
| 5.1.4 | State of control: A condition in which the set of controls consistently provides assurance of continued process performance and product quality. |  |
| 5.1.5 | Control Strategy: A planned set of controls, derived from current product and process understanding that ensures process performance and product quality. The controls can include parameters and attributes related to drug substance and drug product materials and components, facility and equipment operating conditions, in-process controls, finished product specifications, and the associated methods and frequency of monitoring and control. |  |
| 5.2 | Procedure - Continued Process Verification |  |
| 5.2.1 | Process Validation shall be performed in three stages as: - First Stage: Process Design - Second Stage: Process Qualification - Third Stage: Continued Process Verification |  |
| 5.3 | Continued Process Verification |  |
| 5.3.1 | During this stage continuous monitoring of process parameters and quality attributes at the level established during the process qualification stage shall be done. |  |
| 5.3.2 | Ongoing assurance gained during routine production that the process remains in a state of control. |  |
| 5.3.3 | The goal of the third validation stage is continual assurance that the process remains in a state of control (the validated state) during commercial manufacture. |  |
| 5.3.4 | Continued Process Verification shall be performed in steps as given below:- - Identification of CPP (Critical Process Parameters) and CQA (Critical Quality Attributes) for Continued Process Verification. - Continued Process Verification throughout the life cycle of the drug product. - Annual Product Review Product Quality Review (SOP for APQR). |  |
| 5.3.4.1 | Identification of CPP (Critical Process Parameters) and CQA (Critical Quality Attributes) for Continued Process Verification. |  |
| 5.3.4.1.1 | In case of new drug products, in Process Qualification report parameters for Continued Process Verification shall be summarized and tool/methodology to be followed for each parameter throughout the product life cycle shall be given. |  |
| 5.3.4.1.2 | In case of existing commercialized drug products in Continued Process Verification protocol parameters for continued Process Verification and tool/methodology to be followed for each parameter throughout the product life cycle shall be given. |  |
| 5.3.4.2 | In Continued Process Verification monitoring of following parameters shall be performed:- - In-process analysis test results at Bulk Stage (Testing by QC) - Finished Product analysis tests - In-process analysis test performed by production during manufacturing of the batch - Critical Process Parameters during manufacturing of the batch - Yield trends |  |
| 5.3.4.2.1 | In Continued Process Verification only Critical Quality Attributes and variable numerical Critical Process Parameters shall be monitored. |  |
| 5.3.4.3 | Continued Process Verification throughout the life cycle of the drug product: The CPPs and CQAs identified in Continued Process Verification Protocol shall be trended and verified on an ongoing basis during the manufacture of commercial batches. |  |
| 5.3.4.3.1 | The limits for continued process verification shall be finalized after minimum 20 commercial batches by studying the trend and applying scientific rational. |  |
| 5.3.4.3.2 | For existing products last minimum 20 commercial batches shall be considered in deciding parameters for Continued Process Verification. |  |
| 5.3.4.3.3 | Production personnel shall enter the values of critical process parameters whereas Quality control personnel shall enter the values of critical quality attributes in worksheet. Note: Critical quality attributes includes quantifiable results of all test items as per final specification. |  |
| 5.3.4.3.4 | Quality Assurance shall prepare protocol & shall co-ordinate with Production and QC for compilation of data & prepare report. |  |
| 5.3.4.3.5 | Data shall be trended. |  |
| 5.3.4.3.6 | Values of standard deviation and mean identified with minimum 20 batch data shall be entered in file. |  |
| 5.3.4.3.7 | Print outs of such trend charts shall be taken on quarterly basis or after every 20 batches whichever is later. |  |
| 5.3.4.4 | Annual Product Quality Review (APQR): |  |
| 5.3.4.4.1 | During APQR preparation data from all the batches manufactured throughout the year shall be re-evaluated to ensure the manufacturing process is operating in a repeatable reliable fashion and in a state of control. |  |
| 5.3.4.4.2 | During APQR preparation, appropriateness of the current approved control strategy will be confirmed so as to highlight any trends and identify the need for product and/or process improvements where such need exists. |  |
| 5.3.4.4.3 | Data gathered during this stage shall be used to improve and/or optimize the process by altering some aspect of the process of product. |  |
| 5.3.4.4.4 | Based on the data trends, control limits for yields and critical quality attributes may be re-defined with scientific rational by taking change control. |  |
| 5.4 | Continued Process Verification Tools: |  |
| 5.4.1 | Continued Process Verification can be done by many tools and methodologies. Some of them are listed below:- - Control charts. - Process Capability indices calculation |  |
| 5.4.2 | The process capability analysis assesses the process performance relative to the product specification. |  |
| 5.4.3 | Control charts: |  |
| 5.4.3.1 | Control charts also known as Shewhart charts are tools used to determine whether a manufacturing process is in a state of statistical control. |  |
| 5.4.3.2 | It shows the value of the quality characteristic versus the sample number or versus time. |  |
| 5.4.3.3 | In general, the chart contains a center line that represents mean value for the process or limit. |  |
| 5.4.3.4 | Two horizontal lines, called the upper control limit (UCL) and the lower control limit (LCL). |  |
| 5.4.4 | Process Capability indices calculation: Process shifted index / Process capability index (Cpk): |  |
| 5.4.4.1 | It is calculated as, CpK=MIN (Upper Specification Limit – mean)/3 X Standard Deviation, (mean - Lower Specification Limit) 3 x Standard Deviation |  |
| 5.4.4.1.1 | Cpk value more than 1.33 or higher is a capable process. |  |
| 5.5 | Continued Process Verification Protocol & Report shall prepared as per Annexure-I & Annexure-II. |  |
| 5.6 | Each Continued Process Verification Protocol & Report shall be numbered as CPVP/XXYY/BB/AA, CPVR/XXYY/BB/AA Where, CPVP Stands for Continued Process Verification Protocol, CPVR Stands for Continued Process Verification Report, XX Stands for short form of the product name like NH for Naphazoline Hydrochloride, YY Stands for Dosage form like ED Stands for Eye Drops & EO Stands for Eye Ointment, BB Stands for Serial Number of protocol & report start with 01, AA Stands for the Version number start with 00. |  |
| 6.0 | ANNEXURES:- |  |
| 6.1 | Annexure-I : Continued Process Verification Protocol |  |
| 6.2 | Annexure-II : Continued Process Verification Report |  |
| 7.0 | REFERENCES:- |  |
| 7.1 | Guidance for Industry, Process Validation: General Principles and Practices, USFDA, January 2011, Revision 1. |  |
| 7.2 | Guideline on Process Validation European Medicines Agency, Draft, March 2012. |  |
| 8.0 | REVISION HISTORY:- |  |
| Sr. No.; Revision No.; Date of Revision; SOP No.; Supersedes No.; Reason for Revision. 01; 29/12/2022; QAGE132-01; QAGE132-0; - Change in Firm Name from “Indiana Ophthalmics” to “Indiana Ophthalmics LLP”. - Addition of Scope, Annexures, References and Revision History as Section 2.0, 6.0, 7.0 and 8.0 respectively in the SOP.. |