| 1.0 | OBJECTIVE : | To lay down the procedure to determine Total Bacterial Count and Total Fungal Count in raw material. |
| --- | --- | --- |
| 2.0 | SCOPE : | This procedure is applicable for determination of Total Bacterial Count and Total Fungal Count in raw materials. |
| 3.0 | ## RESPONSIBILITY : | QC Microbiologist / QC Head |
| 4.0 | ### ACCOUNTABILITY : | QA Head |
| 5.0 | PROCEDURE : |  |
| 5.1 | Sample Preparation for Microbial Limit Test (MLT) :- |  |
| 5.1.1 | Water Soluble Product :- Dissolve 10 gm or dilute 10 ml of sample (1 in 10 dilution), unless otherwise specified, in sterile 90 ml peptone water or buffered sodium chloride-peptone solution pH 7.0. |  |
| 5.1.2 | Product Insoluble in Water :- Dissolve 10 gm or dilute 10 ml of sample (1 in 10 dilution), unless otherwise specified, in sterile 90 ml peptone water with 0.1% polysorbate 80 or buffered sodium chloride-peptone solution pH 7.0. |  |
| 5.1.3 | Fatty Product :- Dissolve 10 gm or dilute 10 ml of sample (1 in 10 dilution), unless otherwise specified, with 5 gm of polysorbate 80. If necessary heat to not more than 400C, mix carefully while maintaining temperature on the water bath. Add 85 ml of sterile peptone water or buffered sodium chloride-peptone solution pH 7.0. |  |
| 5.1.4 | If further dilution is required prepare with the same diluents used as above. |  |
| 5.2 | Microbial Enumeration Test by Pour Plate Method :- |  |
| 5.2.1 | Total Aerobic Microbial Count :- |  |
| 5.2.1.1 | Aseptically transfer 1 ml sample from 1:10 solution / suspension, prepared as per step 5.1, into two sterile petri plates. |  |
| 5.2.1.2 | Add approximately 15-20 ml of sterile Soybean Casein Digest Agar, prepared as per SOP No. QAMI18 (cooled approximately up to 450C) in both the petri plates. |  |
| 5.2.1.3 | Mix the contents of petri plates by gently swirling the plates for proper mixing of sample and allow the medium to solidify, taking precaution of not spilling the media from the plate. |  |
| 5.2.1.4 | Simultaneously perform the negative control by plating 1 ml of the diluent instead of the sample. |  |
| 5.2.1.5 | Incubate the plates at 30o to 35oC for 3 - 5 days in inverted position. |  |
| 5.2.2 | Total Combined Yeast and Mould Count :- |  |
| 5.2.2.1 | Aseptically transfer 1 ml sample from 1:10 solution / suspension, prepared as per step 5.1, into two sterile petri plates. |  |
| 5.2.2.2 | Add approximately 15-20 ml of sterile Sabouraud Dextrose Agar, prepared as per SOP No. QAMI18 (cooled approximately up to 450C) in both the petri plates. |  |
| 5.2.2.3 | Mix the contents of petri plates by gently swirling the plates for proper mixing of sample and allow the medium to solidify, taking precaution of not spilling the media from the plate. |  |
| 5.2.2.4 | Simultaneously perform the negative control by plating 1 ml of the diluent instead of the sample. |  |
| 5.2.2.5 | Incubate all the plates at 20o to 25oC for 5 days in inverted position. |  |
| 5.3 | Microbial Enumeration Test by Membrane Filtration Method :- |  |
| 5.3.1 | Determine the Total Aerobic Microbial Count and Total Combined Yeast and Mould Count by filtration method. |  |
| 5.3.2 | Use membrane filter having a normal pore size not greater than 0.45µ. Cellulose Nitrate filters / Nylon 66 filters may use for aqueous, oily and weakly alcoholic solutions and cellulose acetate filters may be used for strongly alcoholic. |  |
| 5.3.3 | Aseptically transfer 1 ml sample from 1:10 solution / suspension, prepared as per step 5.1, into each of two membrane filters and filter immediately. |  |
| 5.3.4 | Wash each filter with three portions of about 100 ml of 0.1% peptone water. |  |
| 5.3.5 | Transfer one membrane filter on the surface of pre-incubated Soyabean casein digest agar plate for Total aerobic microbial count and other membrane filter on the surface of Sabouraud dextrose agar plate for total combined yeast and Mould count. |  |
| 5.3.6 | Simultaneously perform the negative control by filtering 100 ml of diluent used instead of sample for each medium. |  |
| 5.3.7 | Incubate the plates of Soyabean casein digest agar at 30-35oC for 5 days for total aerobic microbial count and Sabouraud Dextrose Agar plate at 20-25oC for 5 days for total combined yeast and Mould count in an upright position. |  |
| 5.4 | Observation and Expression of Result :- |  |
| 5.4.1 | After completion of incubation period observe the plates and express the result as Colony Forming Unit (CFU) per gm or ml, by multiplying an average number of CFU/plate with dilution factor. If no colonies are observe express the result as a number of colonies less than dilution factor. Note down the result in Annexure-I (Microbial Limit Test). |  |
| 5.4.2 | If colonies of fungi are detected on Soyabean casein digest agar, they are counted as a part of TAMC and If colonies of bacterial are detected on Sabouraud dextrose agar, they are counted as a part of TYMC. |  |
| 5.4.3 | If the counting of colonies is not possible due to high count or merged colonies, total count test shall be repeated with further dilution steps (Example:- 1:10 and so on) to the solution / suspension. |  |
| 5.4.4 | The negative control plates should not show any growth. Any failed negative control result should be investigated. |  |
| 5.4.5 | If TAMC and TYMC observed are not conforming to specified limits, follow the SOP No. QAMI33 (Handling of out of specification). |  |
| 5.5 | Pathogen testing shall be carried out as per SOP No. QAMI02 (Test for Specified Micro-organisms). |  |
| 6.0 | ANNEXURES:- |  |
| 6.1 | Annexure-I : Microbial Limit Test |  |
| 7.0 | REFERENCES:- |  |
| 7.1 | USP <61> Microbiological Examination |  |
| 7.2 | IP - 2.2.9, Microbial Contamination in Non-sterile Products. |  |
| 8.0 | REVISION HISTORY:- |  |
| Sr. No.; Revision No.; Date of Revision; SOP No.; Supersedes No.; Reason for Revision. 06; 09/01/2023; QAMI01-06; QAMI01-5; - Change in Firm Name from “Indiana Ophthalmics” to “Indiana Ophthalmics LLP”. - Addition of Scope, Annexures, References and Revision History as Section 2.0, 6.0, 7.0 and 8.0 respectively in the SOP.. |