| 1.0 | OBJECTIVE : | To lay down the procedure for specified microorganism in different samples to confirm their absence in the material. |
| --- | --- | --- |
| 2.0 | SCOPE : | This procedure is applicable for test of specified micro-organism in microbiology lab of Indiana Ophthalmic LLP. |
| 3.0 | ## RESPONSIBILITY : | QC Microbiologist, Microbiology in-charge |
| 4.0 | ### ACCOUNTABILITY : | QA Head |
| 5.0 | PROCEDURE : |  |
| 5.1 | Sample Preparation for Specified Microorganisms :- |  |
| 5.1.1 | Prepare the sample for Specified Microorganisms as mentioned in SOP No. QAMI01 (Operational Procedure for Microbial Limit Test). |  |
| 5.1.2 | Transfer the 10 ml prepared sample in 100 ml of sterile Soybean Casein Digest Medium for specified microorganism. |  |
| 5.1.3 | Incubate Soyabean Casein Digest Medium at 30 to 35°C for 18 to 24 hours. |  |
| 5.1.4 | Proceed for Isolation and Identification of Staphylococcus aureus, Pseudomonas aeruginosa, Salmonella, Escherichia coli, Shigella, Candida albicans, Burkholderia cepacia and Bile - Tolerant Gram - Negative Bacteria from this broth. |  |
| 5.2 | Staphylococcus aureus :- |  |
| 5.2.1 | After incubation, shake the tube of Soybean Casein Digest Medium; take a loopfull content from Soyabean casein digest medium tube by using sterile loop and streak in zig - zag pattern until a 1/3 of the plate of Mannitol salt agar is covered. Do not dig into the agar. |  |
| 5.2.2 | Invert and incubate at 30 - 35°C for 18 to 72 hours. After incubation, observe the plate. |  |
| 5.2.3 | The presence of Staphylococcus aureus is indicated by the growth of Yellow or White colonies with Yellow zones. |  |
| 5.2.4 | If colonies are found confirming to the above descriptions, then identification shall be performed by Coagulase test as per following :- |  |
| 5.2.5 | Coagulase Test :- |  |
| 5.2.5.1 | Transfer representative suspected colony from the agar surface of Mannitol Salt Agar to tube, containing 0.5 ml of mammalian, preferably rabbit or horse plasma with or without additives. |  |
| 5.2.5.2 | Incubate at 30 - 35°C. Observe the tube after 3 hours and subsequently at suitable interval up to 24 hours. |  |
| 5.2.5.3 | If no coagulation in any degree is observed, the sample meet the requirement of the test for the absence of Staphylococcus aureus. |  |
| 5.2.5.4 | Record the results in Annexure-I (Test for Specified Microorganism). |  |
| 5.3 | Pseudomonas aeruginosa :- |  |
| 5.3.1 | After incubation, Shake the tube of Soybean Casein Digest Medium; take a loopfull contain from Soyabean casein digest medium tube by using sterile loop and streak in Zig - zag pattern until a 1/3 of the plate of Cetrimide Agar is covered. Do not dig into the agar. |  |
| 5.3.2 | Invert and incubate at 30 - 35°C for 18 to 72 hours. After incubation, observe the plate. |  |
| 5.3.3 | The presence of Pseudomonas aeruginosa is indicated by the growth of greenish color colonies fluorescence in UV light. |  |
| 5.3.4 | If colonies are found confirming to the above descriptions, then identification shall be performed by Pigment test and Oxidase test as per following :- |  |
| 5.3.5 | Pigment Test :- |  |
| 5.3.5.1 | Streak representative suspect colonies from the agar of Cetrimide agar on the surface of Pseudomonas agar medium for detection of fluorescein. |  |
| 5.3.5.2 | Cover and invert the inoculated plates. Incubate at 30 - 35°C for not less than 3 days. |  |
| 5.3.5.3 | Observe the streaked surface area under UV light and indicated by the growth of Yellowish colonies. |  |
| 5.3.5.4 | If colonies are found confirming to above descriptions, then Oxidase test shall be performed to confirm identification as per following :- |  |
| 5.3.6 | Oxidase Test :- |  |
| 5.3.6.1 | If the growth of suspected colonies occurs, place 2 or 3 drops of freshly prepared 1% w/v solution of N, N, Nl, Nl - tetramethyl - 4 - phenylenediamine dihydochloride on filter paper and smear with the suspected colony. |  |
| 5.3.6.2 | If a purple colour is produced within five to ten seconds, the presence of Pseudomonas aeruginosa is confirmed. |  |
| 5.3.6.3 | Record the results in Annexure-II (Test for Specified Microorganism). |  |
| 5.4 | Salmonella :- |  |
| 5.4.1 | After incubation, mix the Soyabean Casein Digest Medium, and transfer 0.1 ml using sterile pipette into 10 ml of Rappaport Vassiliadis Salmonella Enrichment Broth. |  |
| 5.4.2 | Incubate at 30 - 35°C for 18 to 48 hours. |  |
| 5.4.3 | After incubation, mix the content of Rappaport Vassiliadis Salmonella Enrichment Broth and streak a loopful from Rappaport Vassiliadis Salmonella Enrichment Broth on the surface of Xylose Lysine Deoxycholate Agar. |  |
| 5.4.4 | Invert the plates and incubate at 30 - 35°C for 24 to 48 hours. |  |
| 5.4.5 | After incubation, observe the plate, (well developed, red colonies with or without black centers). |  |
| 5.4.6 | If colonies are found confirming to the above description, then Identification test shall be performed as per following:- |  |
| 5.4.6.1 | Individually transfer the suspected colony in Triple Sugar Iron Agar Slant by first inoculating the surface of the slope and then making a stab with the wire loop. |  |
| 5.4.6.2 | At the same time inoculate a tube of urea broth. Incubate at 36 - 38°C for 18 to 24 hours. |  |
| 5.4.6.3 | After incubation, observe the tube of Triple Sugar Iron Agar Medium for the presence of microbial growth and for the following Physical characteristics :- |  |
| 5.4.6.3.1 | Slant Surface :- Alkaline reaction (red colour). |  |
| 5.4.6.3.2 | Butt :- Acid reaction (yellow colour) and / or gas bubble (with or without concomitant blackening). |  |
| 5.4.6.4 | If the butt, slant of Triple Sugar Iron Agar shows growth and physical characteristics confirming to the above descriptions, and red colour is absent in urea broth the presence of Salmonella species is indicated. |  |
| 5.4.6.5 | Record the results in Annexure-III (Test for Specified Microorganism). |  |
| 5.5 | Escherichia coli :- |  |
| 5.5.1 | After incubation, mix the Soyabean Casein Digest Medium, and transfer 1.0 ml using sterile pipette into 100 ml of Macconkey Broth tubes. |  |
| 5.5.2 | Incubate at 42 - 44°C for 18 to 48 hours. |  |
| 5.5.3 | After incubation, mix the content of Macconkey Broth and streak a loopful from Macconkey Broth on the surface of Macconkey Agar plate. |  |
| 5.5.4 | Invert the plates and incubate at 30 - 35°C for 18 to 72 hours. |  |
| 5.5.5 | After incubation, observe the plate, Growth of pink, non-mucoid colonies. |  |
| 5.5.6 | If colonies are found confirming to the above description, then Identification test shall be performed as per following:- |  |
| 5.5.6.1 | Streak representative suspect colonies from the agar of Macconkey Agar on the surface of Levine Eosin Methyl Blue Agar. |  |
| 5.5.6.2 | Invert the plates and incubate at 30 - 35°C for 24 hours. |  |
| 5.5.6.3 | After incubation, observe the plate, Colonies with characteristic of metallic sheen under reflected light and blue black appearance under transmitted light. |  |
| 5.5.10 | Indole Production Test :- |  |
| 5.5.10.1 | Inoculate loop-full culture to 1% Tryptone water and incubate at 30-35°C for 24 hours. |  |
| 5.5.10.2 | After incubation, add 0.5 ml of Kovac's reagent slowly through inner wall of test tube to the surface of the medium. |  |
| 5.5.10.3 | Pink ring observed within l min. |  |
| 5.5.10.4 | Record the results in Annexure-IV (Test for Specified Microorganism). |  |
| 5.6 | Shigella :- |  |
| 5.6.1 | After incubation, mix the Soyabean Casein Digest Medium, and transfer 1.0 ml using sterile pipette into 100 ml of GN Broth. |  |
| 5.6.2 | Incubate at 30 - 35°C for 24 to 48 hours. |  |
| 5.6.3 | After incubation, mix the content of GN broth and streak a loopful from GN broth on the surface of Xylose Lysine Deoxycholate Agar. |  |
| 5.6.4 | Invert the plates and incubate at 30 - 35°C for 24 to 48 hours. |  |
| 5.6.5 | After incubation, observe the plate. The presence of Shigella is indicated by the growth of red colored translucent colony without black center. |  |
| 5.6.6 | Record the results in Annexure-V (Test for Specified Microorganism). |  |
| 5.7 | Candia albicans :- |  |
| 5.7.1 | After incubation, shake the tube of Soybean Casein Digest Medium; take a loopfull content from Soyabean casein digest medium tube by using sterile loop and streak in zig - zag pattern until a 1/3 of the plate of Sabouraud Dextrose Agar is covered. |  |
| 5.7.2 | Invert and Incubate at 30 - 35°C for 24 to 48 hours. |  |
| 5.7.3 | After incubation, observe the plate. The presence of Candida albicans is indicated by the growth of cream coloured colonies. |  |
| 5.7.4 | Record the results in Annexure-VI (Test for Specified Microorganism). |  |
| 5.8 | Burkholderia cepacia :- |  |
| 5.8.1 | After incubation, Shake the tube of Soybean Casein Digest Medium; take a loopfull contain from Soyabean casein digest medium tube by using sterile loop and streak in Zig - zag pattern until a 1/3 of the plate of Burkholderia Cepacia selective Agar is covered. Do not dig into the agar. |  |
| 5.8.2 | Invert and incubate at 30 - 35°C for 18 to 72 hours. After incubation, observe the plate. |  |
| 5.8.3 | The presence of Burkholderia Cepacia is indicated by the growth of greenish-brown colonies with yellow halos, or white colonies surrounded by pink-red zone. |  |
| 5.8.4 | Record the results in Annexure-VII (Test for Specified Microorganism). |  |
| 5.9 | Bile - Tolerant Gram - Negative Bacteria :- |  |
| 5.9.1 | Preparation of Sample and Incubation :- |  |
| 5.9.1.1 | Using Soyabean Casein Digest Medium as diluent, make 1 in 10 dilution of more than 1 gm of the product. |  |
| 5.9.1.2 | Mix well and keep at 20 - 25°C for about 2 to 5 hours to resuscitate the organisms. |  |
| 5.9.2 | Test for Detection of Organisms :- |  |
| 5.9.2.1 | From above prepared sample, take a volume corresponding to 1 gm of the product and inoculate in Enterobactria Enrichment Broth - Mossel. |  |
| 5.9.2.2 | Incubate at 30 - 35°C for 24 to 48 hours. |  |
| 5.9.2.3 | After incubation, mix well content and streak a loopful on surface of Violet Red Bile Glucose Agar. |  |
| 5.9.2.4 | Incubate at 30 - 35°C for 18 to 24 hours. |  |
| 5.9.2.5 | After incubation, observe the plate. Growth of well-developed reddish colonies of Gram Negative Bacteria is considered as positive. |  |
| 5.9.3 | Quantitative Evaluation :- |  |
| 5.9.3.1 | From the above mentioned prepared sample take a volume corresponding to 0.1 gm, 0.01 gm and 0.001 gm (or 0.1 ml, 0.01 ml and 0.001 ml) of product in suitable quantity of Enterobacteria Enrichment Broth - Mossel. |  |
| 5.9.3.2 | Incubate at 30 - 35°C for 24 to 48 hours. |  |
| 5.9.3.3 | Subculture each of the cultures on a plate of Violet Red Bile Glucose Agar. |  |
| 5.9.3.4 | Incubate at 30 - 35°C for 18 to 24 hours. |  |
| 5.9.3.5 | After incubation, observe the plate. Growth of well-developed reddish colonies of gram negative is considered as positive. Record the results in Annexure-VII (Test for Specified Microorganism). |  |
| 5.9.3.6 | Note :- Smallest quantity of the product that gives the positive result and the largest quantity that gives the negative result. The most probable number of bacteria shall be determined from below Table-1. |  |
|  | Table-1 |  |
|  | Result for each Quantity of Product; Probable Number of Bacteria per gm or per ml of Product. 0.1 gm or 0.1 ml; 0.01 gm or 0.01 ml; 0.001 gm or 0.001 ml. +; +; +; More than 103. +; +; -; Less than l03 and More than l02. +; -; -; Less than l02 and More than 10. -; -; -; Less than 10. |
| 6.0 | ANNEXURES:- |  |
| 6.1 | Annexure-I : Test for Specified Microorganism (Staphylococcus aureus) |  |
| 6.2 | Annexure-II : Test for Specified Microorganism (Pseudomonas aeruginosa) |  |
| 6.3 | Annexure-III : Test for Specified Microorganism (Salmonella) |  |
| 6.4 | Annexure-IV : Test for Specified Microorganism (Escherichia coli) |  |
| 6.5 | Annexure-V : Test for Specified Microorganism (Shigella) |  |
| 6.6 | Annexure-VI : Test for Specified Microorganism (Candia albicans) |  |
| 6.7 | Annexure-VII : Test for Specified Microorganism (Burkholderia Cepacia) |  |
| 6.8 | Annexure-VIII : Test for Specified Microorganism (Bile - Tolrant Gram - Negative Bacteria) |  |
| 7.0 | REFERENCES:- NA |  |
| 8.0 | REVISION HISTORY:- |  |
| Sr. No.; Revision No.; Date of Revision; SOP No.; Supersedes No.; Reason for Revision. 06; 10/02/2023; QAMI02-06; QAMI02-5; - Change in Firm Name from “Indiana Ophthalmics” to “Indiana Ophthalmics LLP”. - Addition of Scope, Annexures, References and Revision History as Section 2.0, 6.0, 7.0 and 8.0 respectively in the SOP.. 07; 03/12/2023; QAMI02-07; QAMI02-06; - Addition of isolation and identification procedure for new pathogen, Burkholderia cepacia, as step 5.8. - Addition of Annexure-VII for documenting the isolation and identification procedure for new pathogen, Burkholderia cepacia.. |