| 1.0 | OBJECTIVE : | To describe the procedure for preparation and reviewing the trend of various quality attributes of the product or system and to take corrective actions in case of any abnormalities. |
| --- | --- | --- |
| 2.0 | SCOPE : | This procedure is applicable for various trend analysis at Indiana Ophthalmics LLP. |
| 3.0 | ## RESPONSIBILITY : | QA Officer, QA Executive, QA Sr. Executive, QC Chemist, QC Officer, QC Sr. Officer, QC Executive, QC Head, Production Officer, Production Head |
| 4.0 | ### ACCOUNTABILITY : | QA Head |
| 5.0 | PROCEDURE : |  |
| 5.1 | Trend analysis shall be carried out for various quantitative parameters of raw material, finished products, bulk products, water testing results, environmental monitoring results, microbial analysis data, previous batches data, QC analytical data etc. in order to compare results / data between two or more factors. |  |
| 5.2 | Evaluate the trend of various quantitative tests of incoming products and finished products for pH, assay etc. of consecutive batches. |  |
| 5.3 | Plot the graph of various microbiological data like microbial count of water testing, Bio-burden of manufacturing area monitoring, analytical data of microbial limit test, microbial monitoring of different batches etc. |  |
| 5.4 | Maximum and minimum limit of the respective analytical data shall be shown in the graph. |  |
| 5.5 | Draw the lines for target, alert and action limits wherever applicable for ready reference. |  |
| 5.6 | Draw the lines of approval limit and in-house limit. The values should be always within the in-house limits. |  |
| 5.7 | If any abnormalities observed, investigation shall be done in the following areas as applicable:- - Change of raw material / packaging material A.R. No. - Change of raw material / packaging material vendor. - Change in manufacturing formula. - Change in manufacturing process. - Change in manufacturing equipment design. - Change in analytical method / reagents. - Change in frequency of any manufacturing / QC parameters. - Calibration / Validation status of related instrument / equipment. - Stability profile of the product. - Change in environmental condition. - Change in batch size. - Change in primary pack or pack style. - Change in sanitization process / schedule / adherence. - Time taken for batch completion. |  |
| 5.8 | If any cause is identified, collect more data related to the activities for confirmation and initiate corrective and preventive action to control this variation in future. |  |
| 5.9 | Investigation shall be carried out if any abnormal trend observed. |  |
| 5.10 | Trend analysis of pH, LOD and assay of raw materials shall be covered in APQR of the respective product. |  |
| 5.11 | Trend of pH and assay for bulk product shall be covered in APQR of the respective product. |  |
| 5.12 | Trend of pH, assay, uniformity of volume, osmolality etc. for finished product shall be covered in APQR of the respective product. |  |
| 5.13 | Trend analysis of yield at bulk stage and finished stage shall be compared and covered in APQR of the respective product. |  |
| 5.14 | Preparation of Trends for Water Analysis Results :- |  |
| 5.14.1 | The trend for water analysis results shall be prepared for the following type of water:- |  |
|  | Sr. No.; Type of Water; Parameters. 1.; Raw Water; Residue on Evaporation, pH, Conductivity, Total Hardness. 2.; RO Water; Residue on Evaporation, pH, Conductivity, Total Hardness. 3.; DM Water; Residue on Evaporation, pH, Conductivity, Total Hardness. 4.; Distilled Water; Residue on Evaporation, pH, Conductivity. |
| 5.14.2 | The daily acquired data of each parameters as per above table for each type of water analysis shall be taken for trend analysis. |  |
| 5.14.3 | Trend results shall be prepared every month. The monthly report shall contain but not limited to date, sampling point, parameter name with limit and conclusion. |  |
| 5.14.4 | Trend in graphical representation shall be prepared. Each trend analysis report shall be assigned with annexure number and attachment number as per below:- |  |
| 5.14.4.1 | For distilled water used in production:- |  |
|  | Annexure-A :- Distilled Water for Chemical Analysis |  |
|  | Annexure-B :- Distilled Water for Microbiological Analysis (Microbial Count) |  |
|  | Annexure-C :- Distilled Water for Microbiological Analysis (Pathogens) |  |
| 5.14.4.2 | Chemical Analysis:- |  |
|  | Annexure-I :- Raw Water for Chemical Analysis |  |
|  | Annexure-II :- RO Water for Chemical Analysis |  |
|  | Annexure-III :- DM Water for Chemical Analysis |  |
|  | Annexure-IV :- Distilled Water for Chemical Analysis |  |
| 5.14.4.3 | Microbiological Analysis:- |  |
|  | Annexure-I-A :- Raw Water for Microbiological Analysis |  |
|  | Annexure-II-A :- RO Water for Microbiological Analysis |  |
|  | Annexure-III -A:- DM Water for Microbiological Analysis |  |
|  | Annexure-IV-A :- Distilled Water for Microbiological Analysis |  |
| 5.14.4.4 | Graphical Representation for Chemical Analysis:- |  |
|  | Attachment-I :- Graphical Representation of Raw Water for Chemical Analysis |  |
|  | Attachment-II :- Graphical Representation of RO Water for Chemical Analysis |  |
|  | Attachment-III :- Graphical Representation of DM Water for Chemical Analysis |  |
|  | Attachment-IV :- Graphical Representation of Distilled Water for Chemical Analysis |  |
| 5.14.4.5 | Graphical Representation for Microbiological Analysis:- |  |
|  | Attachment-I-A :- Graphical Representation of Raw Water for Microbiological Analysis |  |
|  | Attachment-II-A :- Graphical Representation of RO Water for Microbiological Analysis |  |
|  | Attachment-III-A :- Graphical Representation of DM Water for Microbiological Analysis |  |
|  | Attachment-IV-A :- Graphical Representation of Distilled Water for Microbiological Analysis |  |
| 5.15 | Preparation of Trends for Quality Management System :- |  |
| 5.15.1 | Trends for the quality management systems such as change control, deviation, CAPA, OOS, market complaints, incidents etc. shall be prepared on yearly basis preferably in the every January month by covering the data of previous entire year. |  |
| 5.15.2 | Data shall be gathered from the relevant QMS documents. Ensure that data is accurate and complete. |  |
| 5.15.3 | Trend analysis of change control shall include department name vs. total number of change control, Type of change control (document related or facility related or others) vs. total number of change control, Type of change control (critical, major and minor) vs. total number of change control, Status of change control (approved or rejected) vs. total number of change control, status of change control (open or closed) vs. total number of change control, etc. |  |
| 5.15.4 | Trend analysis of deviation shall include department name vs. number of deviation, Type of deviation (product, material or system related) vs. total number of deviation, Type of deviation (open or closed) vs. total number of deviation, etc. |  |
| 5.15.5 | Trend analysis of CAPA shall include department name vs. number of CAPA, Status of CAPA (open or closed) vs. number of CAPA, etc. |  |
| 5.15.6 | Trend analysis of laboratory incident shall include name of test vs. number of lab incidents, name of analyst vs. number of lab incidents, status of lab incidents (open or closed) vs. number of lab incidents, reason for lab incidents vs. number of lab incidents, etc. |  |
| 5.15.7 | Trend analysis of OOS shall include name of test vs. number of OOS, name of analyst vs. number of OOS, status of OOS (open or closed) vs. number of OOS, etc. |  |
| 5.15.8 | Trend analysis of market complaint shall include type of defect vs. number of market complaint, complaint category vs. number of market complaint, complaint status vs. number of market complaint, etc. |  |
| 5.15.9 | Trend analysis of incident shall include description of incident vs. number of incident, department name vs. number of incident, incident status vs. number of incident, etc. |  |
| 5.15.10 | Trends for Quality Management System shall be prepared as per Annexure-III (Format for Preparation of QMS Trend). |  |
| 5.16 | Handling of OOT (Out Of Trend) :- |  |
| 5.16.1 | This procedure is applicable to the out of trend results observed in the analysis of raw materials, packing materials, finished product, in-process samples, environmental monitoring results, water results and stability samples with respect to previous stability test point conducted. |  |
| 5.16.2 | Out of trend result shall be determined on the basis of the data of previous batches or previous analytical data or aberrant data observed during analysis. The result may be within the specification but not in the trend. |  |
| 5.16.3 | While performing the analysis, if any OOT or aberrant data is observed, then the analyst shall immediately inform to QC head. |  |
| 5.16.4 | QC Head shall conduct the preliminary investigation alongwith the analyst of the OOT observation as per Annexure-II (OOT Investigation Form). |  |
| 5.16.5 | Analyst shall store the sample and standard preparations alongwith any other preparation used during the analysis until the investigation is completed. |  |
| 5.16.6 | Make the necessary entries in the OOT Register as per Annexure-I (OOT Register). |  |
| 5.16.7 | Assign OOT number as per follow:- |  |
|  | OOT-YY-NN |  |
|  | Where, |  |
|  | - OOT is the Out Of Trend, - YY is the last two digit of the current year, - NN is the continuous serial number starting from 01. |  |
| 5.16.8 | During the OOT investigation, if the cause of OOT result is identified as laboratory error, repeat analysis shall be carried out and shall be reported. |  |
| 5.16.9 | The same analyst shall repeat the test with a fresh sample and fresh standard preparation (if required) with due care to omit the errors identified. |  |
| 5.16.10 | In case if the cause of OOT result is not identified as laboratory error, Head QC shall recommend for reanalysis to confirm the OOT results. If the result of repeat analysis is similar to a previous analysis the OOT results are valid and same shall be informed to concerned department. |  |
| 5.16.11 | The analyst shall record the re-analysis results in the respective protocol. |  |
| 5.16.12 | If the re-analysis results are within the trend and observations are not aberrant, then the initial analysis shall be investigated for the cause of OOT and report the repeat analysis results. |  |
| 5.16.13 | If the cause of OOT results are not identified, and the results of re-analysis are same as the previous analysis then the results obtained during the first analysis shall be reported. |  |
| 5.16.14 | If the test results are out of trend, it shall be concluded that the material / product is not meeting the trend and the final decision for acceptance of material / product shall be taken by Head QA. |  |
| 5.16.15 | In the case of raw and packing materials, Head QC shall review the OOT results and forward the same to QA and supplier for investigation and prevent the recurrence of such incident in the future supplies. |  |
| 5.16.16 | In case of finished products, Head QC shall review the OOT results and forward the same to QA and Production for information and investigation. A final decision shall be taken by Head QA based on the investigation report. |  |
| 5.16.17 | In case of stability studies, OOT investigation shall be carried out and if required, analysis can be performed using the samples of the previous stability test point. A final decision shall be taken by the head QA based on the investigation report in co-ordination with production. |  |
| 5.16.18 | If the product belongs to the loan license party, the same shall be informed to loan license party. |  |
| 6.0 | ANNEXURES:- |  |
| 6.1 | Annexure-I : Out of Trend (OOT) Register |  |
| 6.2 | Annexure-II : OOT Investigation Form |  |
| 6.3 | Annexure-III : Format for Preparation of QMS Trend |  |
| 7.0 | REFERENCES:- NA |  |
| 8.0 | REVISION HISTORY:- |  |
| Sr. No.; Revision No.; Date of Revision; SOP No.; Supersedes No.; Reason for Revision. 05; 25/09/2022; QAGE72-05; QAGE72-4; - Change in Firm Name from “Indiana Ophthalmics” to “Indiana Ophthalmics LLP”. - Addition of Annexures, References and Revision History as Section 6.0, 7.0 and 8.0 respectively in the SOP.. 06; 17/10/2025; QAGE72-06; QAGE72-05; - Addition of Scope as section 2.0 in the SOP. - Addition of section 5.15 as Preparation of Trends for Quality Management System. - Addition of Annexure-III as Format for Preparation of QMS Trend.. |