| 1.0 | OBJECTIVE : | To lay down the procedure for conducting Stability Studies of Finished Products. |
| --- | --- | --- |
| 2.0 | SCOPE : | This procedure is applicable for stability study of finished products to be conducted as Indiana Ophthalmics LLP. |
| 3.0 | RESPONSIBILITY : | IPQA Officer, QA Executive, QA Sr. Executive, QC Chemist, QC Officer, QC Executive, QC Manager, QA Head |
| 4.0 | ACCOUNTABILITY : | QA Head |
| 5.0 | PROCEDURE : |  |
| 5.1 | Definition:- |  |
| 5.1.1 | Stability:- |  |
| 5.1.1.1 | The ability of a pharmaceutical product to retain its properties within specified limits throughout its shelf life. |  |
| 5.1.2 | Stability Sample:- |  |
| 5.1.2.1 | Sufficient number of samples to be retained for carrying out stability study depending upon the number of tests to be performed at each checkpoint. |  |
| 5.1.2.2 | Stability samples should be stored in the designated area under specified storage conditions. Temperature and Humidity conditions should be monitored continuously. |  |
| 5.2 | Purpose of Stability Studies:- |  |
| 5.2.1 | To select adequate formulations and container closure systems for product development through Accelerated stability study. |  |
| 5.2.2 | To determine shelf life and storage conditions for product through accelerated and long term stability study. |  |
| 5.2.3 | To substantiate the claimed shelf life for registration dossier through long term stability study. |  |
| 5.2.4 | To verify that no changes have been introduced in the formulation or manufacturing process that can adversely affect the stability of the product and for assurance of product quality through Accelerated as well as Long Term Stability Study. |  |
| 5.3 | Stability Environmental Conditions:- |  |
| 5.3.1 | The samples shall be stored at 40°C ± 2°C / 75 % ± 5 % RH for accelerated stability study testing. The testing frequency for accelerated stability study shall be 0 (initial), 1, 3 and 6 months. |  |
| 5.3.2 | The samples for long term stability study shall be stored at 30°C ± 2°C / 75% ± 5% RH. The testing frequency for long term stability study shall be 0 (initial), 3, 6, 9, 12, 18, 24, 30 and 36 months and so on. |  |
| 5.3.3 | For the products intended for storage in a refrigerator, the stability study shall be performed by considering storage condition as 5⁰C ± 3°C for long-term stability analysis and as 30°C ± 2°C / 75 % ± 5 % RH for accelerated stability study. |  |
| 5.3.4 | Stability samples must be stored in chambers. Exposure to light caused by entering and withdrawing samples and by technical control or service activities are allowed. |  |
| 5.3.5 | The actual temperature and humidity should be monitored during stability storage. The effect of excursion due to equipment failure should be addressed and reported if judged to affect stability results. |  |
| 5.3.6 | Excursion that exceeds the define tolerance for more than 24 hours should be describe in the study report and their effect shall be assessed. |  |
| 5.4 | Selection of Batches:- |  |
| 5.4.1 | Validation Batches:- |  |
| 5.4.1.1 | These are the batches after commercialization of a new product. The stability study to be done on first three batches of same formulation and dosage form in the Container or Closure proposed for marketing, for long term and accelerated stability study. |  |
| 5.4.1.2 | In case of change in MOC of primary packing material of a product, long term as well accelerated stability study shall be carried out for three batches. |  |
| 5.4.2 | Commercial Batches:- |  |
| 5.4.2.1 | For commercial batches, at least one batch per year of product manufactured in every strength and every primary packaging type should be included in the stability programme. |  |
| 5.5 | Sampling, Charging and Withdrawal of Stability Samples:- |  |
| 5.5.1 | The IPQA (In-process Quality Assurance) officer shall collect the product samples for stability studies from process validation batches and subsequently at least one batch per year of product manufactured in every strength and every primary packaging type. |  |
| 5.5.2 | QA personnel shall store the stability samples into the designated storage conditions (i.e. accelerated stability and long term stability) within the respective stability chambers. Simultaneously, QA personnel shall document this action by making the required entries in Annexure-I (Accelerated Stability Sample Register) and Annexure-II (Long Term Stability Sample Register). |  |
| 5.5.3 | Each individual stability sample bottle or tube must be individually labeled with its intended storage condition. The sample intended for long-term stability study shall contain a label indicating its storage condition as “30°C/75%RH” and the sample intended for accelerated stability study shall contain a label indicating its storage condition as “40°C/75%RH”. This ensures quick and accurate identification. |  |
| 5.5.4 | The entire stability sample bag (containing long-term and accelerated samples) shall be labelled as per Annexure-VII (Stability Sample Bag Label). This label shall include the details of stability condition (either long-term or accelerated), product name, batch number, mfg. date, exp. date, quantity and date. |  |
| 5.5.5 | The samples shall be placed under storage conditions within the 30 calendar days after the date of product release. |  |
| 5.5.6 | The product release tests results shall serve as the initial (Zero-time) stability results. If the Product batch is not placed on stability within 30 calendar days after the date of release test, then full product testing must be conducted again to establish the initial (Zero-time) stability tests results. |  |
| 5.5.7 | Withdrawal of the stability samples shall be completed for analysis within time period (or +8 days allowable). QA person shall withdraw the stability samples as per schedule and handover to QC person. QC person must complete the analysis within 25 working days from the date of withdrawal. |  |
| 5.5.8 | Stability sample shall be withdrawn on weekly basis (+8 days allowable). For example, if sample charge date is 02/06/2025 then, |  |
|  | - 1st month withdrawal time shall be from 02/07/2025 to 09/07/2025, |  |
|  | - 3rd month withdrawal time shall be from 02/09/2025 to 09/09/2025, |  |
|  | - 6th month withdrawal time shall be from 02/12/2025 to 09/12/2025. |  |
| 5.5.9 | Before starting of analysis, sample shall attain the room temperature. |  |
| 5.5.10 | After withdrawal of stability samples, sample shall keep in designated area under lock & key in proper temperature. |  |
| 5.5.11 | For any deviation from the above testing time range, separate report shall be generated covering the risk assessment of the deviation. |  |
| 5.6 | Bracketing of Product:- |  |
| 5.6.1 | In case the product has identical formulation but there are different packing configurations with same primary packing material, then any one type of packing configuration shall be chosen and samples shall be collected. |  |
| 5.6.2 | If product is manufactured under different brand names but the formulation, strength and manufacturing process remains same with same primary packing materials then samples of any one brand name shall be collected. |  |
| 5.7 | Protocol, Report and Review:- |  |
| 5.7.1 | Stability study protocol shall be prepared by QC personnel, reviewed by QC Manager and approved by QA Head. |  |
| 5.7.2 | Accelerated stability study protocol numbering shall be assigned as:- |  |
|  | ASP/XXXXX/FF/NNN |  |
|  | Accelerated stability study report numbering shall be assigned as:- |  |
|  | ASR/XXXXX/FF/NNN |  |
|  | Where, - ASP stands for Accelerated Stability Protocol, - ASR stands for Accelerated Stability Report, - XXXXX stands for product identification number (Number of characters may vary depending upon the product code), - FF stands for formulation type in two digits, i.e., ED for eye drops, EO for eye ointment, EP for External Preparation, VP for Veterinary Product, IO stands for Intra-mammary Ointment, - NNN stands for continuous serial number starts from 001. |  |
| 5.7.3 | Long-term stability study protocol numbering shall be assigned as:- |  |
|  | SSP/XXXXX/FF/NNN |  |
|  | Long-term stability study report numbering shall be assigned as:- |  |
|  | SSR/XXXXX/FF/NNN |  |
|  | Where, - SSP stands for Long Term Stability Study Protocol, - SSR stands for Long Term Stability Study Report, - XXXXX stands for product identification number (Number of characters may vary depending upon the product code), - FF stands for formulation type in two digits, i.e., ED for eye drops, EO for eye ointment, EP for External Preparation, VP for Veterinary Product, IO stands for Intra-mammary Ointment, - NNN stands for continuous serial number starts from 001. |  |
| 5.7.4 | Stability test protocol shall comprises of but not limited to Objective, Scope, Responsibilities, Test parameters, Details of sample withdrawal, Stability program with sample quantity and Schedule of Study. Stability test protocol for Accelerated & Long term stability study shall be prepared as per Annexure-III (Accelerated Stability Study Protocol) & V (Long Term Stability Study Protocol) respectively. |  |
| 5.7.5 | After completion of the stability study i.e. Accelerated Stability and Long Term Stability Study, QC person shall prepare stability study report as per Annexure-IV (Accelerated Stability Study Report) & VI (Long Term Stability Study Report) respectively. QC Manager shall thoroughly review the report & submit it to QA department. In case of any abnormality, corrective actions to be taken jointly. |  |
| 5.8 | Sample Analysis and Data Evaluation:- |  |
| 5.8.1 | QA person shall withdraw the stability samples as per schedule (or +8 days allowable) and handover to QC person for testing purpose. Tests like Sterility testing, Particle size, Uniformity of volume, Capping, Viscosity, IR and metal particle shall carry out at initial and last stage. Test like Microbial Limit Test (MLT) shall carry out at 1st month and 3rd month interval for accelerated stability study and at 3rd month and 6th month interval for long term stability study. |  |
| 5.8.2 | After completion of analysis, QC analyst shall calculate the results and ensure date and signatures on all generated documents like Chromatograms, Analysis reports and Calculation sheets. |  |
| 5.8.3 | QC analyst shall compile the results of stability analysis in the Stability Study report. |  |
| 5.8.4 | QC Manager or designee shall review the data at the end of the each station of stability study. |  |
| 5.8.5 | Any significant changes observed in stability study of product should inform to QA Head for corrective action. |  |
| 5.8.6 | If the product is manufactured on Loan License basis, the stability studies should be carried out as per their requirement & any deviation in observation is communicated to Head QA of the loanee firm. |  |
| 5.8.7 | Any deviation in stability testing with respect to stability protocol shall be recorded and shall be assessed by Quality Assurance Head. |  |
| 5.8.8 | Out of specifications (OOS) results are investigated and corrective measures should be taken accordingly. |  |
| 5.8.9 | Based on the stability results obtained and their evaluation an appropriate re-test period, shelf life, storage condition, transport conditions, shall be established and updated. |  |
| 5.9 | Precautions:- |  |
| 5.9.1 | In case of failure of stability chambers, rectification shall be done within 24 hours. |  |
| 5.9.2 | If rectification is not carried out within 24 hrs., in such case sample withdrawal schedule of each station shall be extended as per stability chamber failure time. |  |
| 6.0 | ANNEXURES:- |  |
| 6.1 | Annexure-I : Accelerated Stability Sample Register |  |
| 6.2 | Annexure-II : Long Term Stability Sample Register |  |
| 6.3 | Annexure-III : Accelerated Stability Study Protocol |  |
| 6.4 | Annexure-IV : Accelerated Stability Study Report |  |
| 6.5 | Annexure-V : Long Term Stability Study Protocol |  |
| 6.6 | Annexure-VI : Long Term Stability Study Report |  |
| 6.7 | Annexure-VII : Stability Sample Bag Label |  |
| 7.0 | REFERENCES:- |  |
| 7.1 | ICH Q1A (R2) |  |
| 7.2 | WHO 953 Annex 2 |  |
| 8.0 | REVISION HISTORY:- |  |
| Sr. No.; Revision No.; Date of Revision; SOP No.; Supersedes No.; Reason for Revision. 09; 26/09/2022; QAGE75-09; QAGE75-8; - Change in Firm Name from “Indiana Ophthalmics” to “Indiana Ophthalmics LLP”. - Addition of Annexures, References and Revision History as Section 6.0, 7.0 and 8.0 respectively in the SOP.. 10; 13/07/2023; QAGE75-10; QAGE75-09; - Addition of Scope as Section 2.0 in the SOP. - Addition of Provision for writing reference SOP No. in Annexure-III, Annexure-IV, Annexure-V and Annexure-VI.. 11; 24/11/2023; QAGE75-11; QAGE75-10; - Inclusion of MLT test at 1st month and 3rd month interval for accelerated stability study and at 3rd month and 6th month interval for long term stability study in step 5.8.1.. 12; 10/12/2025; QAGE75-12; QAGE75-11; - Addition of step 5.5.3 and 5.5.4 in the SOP to establish mandatory labeling for individual stability sample bottles/tubes and for the entire stability sample bag, respectively for quick and easy identification. - Addition of Annexure-VII as “Stability Sample Bag Label” to standardize the labeling process.. |