| 1.0 | OBJECTIVE : | To describe the procedure for in-process checks to be performed during manufacturing, filling and packaging operation. |
| --- | --- | --- |
| 2.0 | ## RESPONSIBILITY : | IPQA Personnel, Production Chemist, Production Head |
| 3.0 | ACCOUNTABILITY : | QA Head |
| 4.0 | PROCEDURE : |  |
| 4.1 | In-process checks shall be performed by IPQA personnel and production chemist during different batch processing stages such as manufacturing, filling, packing etc. at defined time intervals and the same shall be recorded in respective batch records. |  |
| 4.2 | The required number of samples shall be taken to verify quality of product during batch processing of drops, ointment and syringe products as per described below:- |  |
| 4.2.1 | In-Process Checks during Manufacturing of Drops Products:- |  |
| 4.2.1.1 | Manufacturing Stage : |  |
| 4.2.1.1.1 | Line clearance of the manufacturing area, equipment and process shall be given as per SOP No. QAGE80 (Line Clearance). |  |
| 4.2.1.1.2 | Check that dispensed materials are added in proper sequence and quantity as per respective batch manufacturing record. |  |
| 4.2.1.1.3 | Verify stirring time with respect to respective batch manufacturing process. |  |
| 4.2.1.1.4 | Verify the pH of the solution with production chemist and record the observation in the respective batch record. |  |
| 4.2.1.1.5 | On receipt of the test request slip from the production personnel, collect the bulk sample for QC analysis. |  |
| 4.2.1.1.6 | Bulk analysis observation shall be recorded in respective BMR. |  |
| 4.2.1.1.7 | Verification of the yield shall be done at the end of the manufacturing stage. |  |
| 4.2.1.2 | Filling Stage : |  |
| 4.2.1.2.1 | Line clearance of the filling area, equipment and process shall be given as per SOP No. QAGE80 (Line Clearance). |  |
| 4.2.1.2.2 | Collect randomly 10 sterile empty bottles from filling line for checking cleanliness of bottles at every 30 minutes ± 10 minutes during filling activity. |  |
| 4.2.1.2.3 | Perform challenge test for filling machine and checkweigher. |  |
| 4.2.1.2.4 | On completion of initial setting of filling machine, check the filled bottles for in-process parameters. |  |
| 4.2.1.2.5 | Bottles shall be taken from each filling nozzle for in-process checks. |  |
| 4.2.1.2.6 | If parameter of filled bottle is within specified limit of BMR, further operation of the filling machine is allowed otherwise the setting shall be rectified accordingly. |  |
| 4.2.1.2.7 | Collect randomly at least 01 filled and sealed bottle from each nozzle of filling machine at every 30 minutes ± 10 minutes time interval for checking below in-process parameters:- |  |
|  | - Filled volume:- By emptying the bottle content in the measuring cylinder. - Sealing quality:- By visual inspection. - Particulate matter:- By verifying the bottle content visually. - Properness of plugging:- By visual inspection. - Properness of capping:- By visual inspection. |  |
| 4.2.1.2.8 | If, during in-process checks, it is found that any parameter of filled bottle is out of the specified limits, as per BMR, the machine shall be stopped. |  |
| 4.2.1.2.9 | The bottles filled between the last check (by production or QA whichever is latest) and current check shall be segregated. |  |
| 4.2.1.2.10 | All the bottles shall be checked for compliance (of in-process parameter) to the specifications as per the BMR. |  |
| 4.2.1.2.11 | The bottles, which are out of specified limit, shall be labeled as “On Hold” and rest of the bottles shall be cleared if all parameter are within specified limits of BMR. |  |
| 4.2.1.2.12 | Head QA shall take the decision on “On Hold” bottles. |  |
| 4.2.1.3 | Visual Inspection Stage : |  |
| 4.2.1.3.1 | In case of opaque plastic bottles, take about 0.2% of filled bottles from various crates and carry out visual inspection by emptying the solution in clean and dry test tube for particulate matter (such as fibers, glass particles, black particles, white particles, foreign particles etc.) and volume against white and black background. If any critical defect observed, then take 1.0 % of filled bottles from the same batch to carry out visual inspection. If any critical defect observed during inspection of these 1.0 % bottles, then hold the batch and do not release until proper investigation is completed. |  |
| 4.2.1.3.2 | In case of transparent plastic bottles, visual inspection of bottles shall be done against white and black background as per procedure mentioned in SOP No. PRPA02 (Visual Inspection of Filled Plastic Bottles and Tubes (Finish Product)). |  |
| 4.2.1.4 | Packaging Stage : |  |
| 4.2.1.4.1 | Line clearance of the packing area, equipment and process shall be given as per SOP No. QAGE80 (Line Clearance). |  |
| 4.2.1.4.2 | In-process checks shall be performed by IPQA personnel and packing supervisor alternatively at every 1 hour ± 15 minutes during packing activity. |  |
| 4.2.1.4.3 | IPQA personnel and packing supervisor shall randomly check approximately 10 nos. of packed bottles from the running packing line at every 1 hour ± 15 minutes. |  |
| 4.2.1.4.4 | Following in-process checks shall be considered by IPQA personnel and packing supervisor during packing activity:- |  |
|  | - Proper labeling on bottles, i.e., availability of label on bottle, proper pasting of label on bottle, product name, generic name, License no. and label claim on label as per BPR, batch coding details on label as per BPR (batch no., mfg. date, exp. Date, MRP etc.). |  |
|  | - Availability of leaflet (if applicable as per BPR) in the packed carton, printing matter on leaflet, product name, label claim etc. on leaflet. |  |
|  | - Proper pasting of printed box, batch coding on printed box as per BPR (batch no., mfg. date, exp. Date, MRP etc.), product name, generic name, License no. and label claim on printed box as per BPR, printing matter on printed box, colour variation (if any) between two or more printed box, flap of carton. |  |
|  | - Leakage of bottles, availability of cap, proper closing of bottle with cap. |  |
|  | - Quality of outer box (if applicable as per BPR), label available on outer box, details on label of outer box, numbers of inner box available in outer box as per BPR, proper shrink quality (if shrink applicable). |  |
|  | - Number of containers in the shipper and their packing arrangement. |  |
|  | - BOPP taping quality. |  |
|  | - Coding details on shipper and on shipper label as per BPR. |  |
| 4.2.1.4.5 | Collect the packed control samples and stability samples or any other required samples during packaging operation. |  |
| 4.2.1.4.6 | Verify the yield reconciliation after the completion of packing. |  |
| 4.2.2 | In-Process Checks during Manufacturing of Ointment and Gel Products:- |  |
| 4.2.2.1 | Manufacturing Stage : |  |
| 4.2.2.1.1 | Line clearance of the manufacturing area, equipment and process shall be given as per SOP No. QAGE80 (Line Clearance). |  |
| 4.2.2.1.2 | Check that dispensed materials are added in proper sequence and quantity as per respective batch manufacturing record. |  |
| 4.2.2.1.3 | Verify stirring time with respect to respective batch manufacturing process. |  |
| 4.2.2.1.4 | If required, check the pH of the ointment / gel with production chemist and record the observation in the respective batch record. |  |
| 4.2.2.1.5 | On receipt of the test request slip from the production personnel, collect the bulk sample for QC analysis. |  |
| 4.2.2.1.6 | Bulk analysis observation shall be recorded in respective BMR. |  |
| 4.2.2.1.7 | Verification of the yield shall be done at the end of the manufacturing stage. |  |
| 4.2.2.2 | Filling Stage : |  |
| 4.2.2.2.1 | Line clearance of the filling area, equipment and process shall be given as per SOP No. QAGE80 (Line Clearance). |  |
| 4.2.2.2.2 | Collect randomly 10 sterile empty tubes from filling line for checking cleanliness of tubes at every 30 minutes ± 10 minutes during filling activity. |  |
| 4.2.2.2.3 | Perform challenge test for filling machine. |  |
| 4.2.2.2.4 | On completion of initial setting of filling machine, check the filled tubes for in-process parameters. |  |
| 4.2.2.2.5 | Tubes shall be taken from each filling nozzle for in-process checks. |  |
| 4.2.2.2.6 | If parameter of filled tube is within specified limit of BMR, further operation of the filling machine is allowed otherwise the setting shall be rectified accordingly. |  |
| 4.2.2.2.7 | Collect randomly at least 01 filled tube from each nozzle of filling machine at every 30 minutes ± 10 minutes time interval for checking below in-process parameters:- |  |
|  | - Filled weight:- By weighing on the weighing balance. - Properness of shape:- By visual inspection. - Crimping:- By visual inspection. - Leakage:- By thumb press and then verifying visually. - Batch Coding:- By visual verification. - Printing matter:- By visual verification. |  |
| 4.2.2.2.8 | If, during in-process checks, it is found that any parameter of filled tube is out of the specified limits, as per BMR, the machine shall be stopped. |  |
| 4.2.2.2.9 | The tubes filled between the last check (by production or QA whichever is latest) and current check shall be segregated. |  |
| 4.2.2.2.10 | All the tubes shall be checked for compliance (of in-process parameter) to the specifications as per the BMR. |  |
| 4.2.2.2.11 | The tubes, which are out of specified limit, shall be labeled as “On Hold” and rest of the tubes shall be cleared if all parameter are within specified limits of BMR. |  |
| 4.2.2.2.12 | Head QA shall take the decision on “On Hold” tubes. |  |
| 4.2.2.3 | Visual Inspection Stage : |  |
| 4.2.2.3.1 | During visual inspection of filled tubes, 100 % tubes shall be checked for leakage (cap side and crimp side), batch coding rejection (light coding, colour peel-off etc.) and appearance (dented tube, tilted tube, eye mark orientation rejection etc.). |  |
| 4.2.2.4 | Packing Stage : |  |
| 4.2.2.4.1 | Line clearance of the packing area, equipment and process shall be given as per SOP No. QAGE80 (Line Clearance). |  |
| 4.2.2.4.2 | In-process checks shall be performed by IPQA personnel and packing supervisor alternatively at every 1 hour ± 15 minutes during packing activity. |  |
| 4.2.2.4.3 | IPQA personnel and packing supervisor shall randomly check approximately 10 nos. of packed tubes from the running packing line at every 1 hour ± 15 minutes. |  |
| 4.2.2.4.4 | Following in-process checks shall be considered by IPQA personnel and packing supervisor during packing activity:- |  |
|  | - Product name, generic name, label claim and other text matter on the tube. |  |
|  | - Proper crimping of tube, leakage of tube, Batch no., mfg. date / exp. date on the crimp of the tube. |  |
|  | - Availability of leaflet (if applicable as per BPR) in the packed carton, printing matter on leaflet, product name, label claim etc. on leaflet. |  |
|  | - Proper pasting of printed box, batch coding on printed box as per BPR (batch no., mfg. date, exp. Date, MRP etc.), product name, generic name, License no. and label claim on printed box as per BPR, printing matter on printed box, colour variation (if any) between two or more printed box, flap of carton. |  |
|  | - Proper sealing and capping of tube. |  |
|  | - Quality of outer box (if applicable as per BPR), label available in outer box, details on label of outer box, numbers of inner box available on outer box as per BPR, proper shrink quality (if shrink applicable). |  |
|  | - Number of containers in the shipper and their packing arrangement. |  |
|  | - BOPP taping quality. |  |
|  | - Coding details on shipper and on shipper label as per BPR. |  |
| 4.2.2.4.5 | Collect the packed control samples and stability samples or any other required samples during packaging operation. |  |
| 4.2.2.4.6 | Verify the yield reconciliation after the completion of packing. |  |
| 4.2.3 | In-Process Checks during Manufacturing of Infusion (Syringes) Products:- |  |
| 4.2.3.1 | Manufacturing Stage : |  |
| 4.2.3.1.1 | Line clearance of the manufacturing area, equipment and process shall be given as per SOP No. QAGE80 (Line Clearance). |  |
| 4.2.3.1.2 | Check that dispensed materials are added in proper sequence and quantity as per respective batch manufacturing record. |  |
| 4.2.3.1.3 | Verify stirring time with respect to respective batch manufacturing process. |  |
| 4.2.3.1.4 | On receipt of the test request slip from the production personnel, collect the bulk sample for QC analysis. |  |
| 4.2.3.1.5 | Bulk analysis observation shall be recorded in respective BMR. |  |
| 4.2.3.1.6 | Verification of the yield shall be done at the end of the manufacturing stage. |  |
| 4.2.3.2 | Filling Stage : |  |
| 4.2.3.2.1 | Line clearance of the filling area, equipment and process shall be given as per SOP No. QAGE80 (Line Clearance). |  |
| 4.2.3.2.2 | Collect randomly 10 sterile empty syringes from filling line for checking cleanliness of syringes at every 30 minutes ± 10 minutes during filling activity. |  |
| 4.2.3.2.3 | Perform challenge test for filling machine. |  |
| 4.2.3.2.4 | On completion of initial setting of filling machine, check the filled syringe for in-process parameters. |  |
| 4.2.3.2.5 | Syringes shall be taken from each filling nozzle for in-process checks. |  |
| 4.2.3.2.6 | If parameter of filled syringe is within specified limit of BMR, further operation of the filling machine is allowed otherwise the setting shall be rectified accordingly. |  |
| 4.2.3.2.7 | Collect randomly at least 01 filled syringe from each nozzle of filling machine at every 30 minutes ± 10 minutes time interval for checking below in-process parameters:- |  |
|  | - Filled weight, - Properness of plunging, - Leakage |  |
| 4.2.3.2.8 | If, during in-process checks, it is found that any parameter of filled syringe is out of the specified limits, as per BMR, the machine shall be stopped. |  |
| 4.2.3.2.9 | The syringe filled between the last check (by production or QA whichever is latest) and current check shall be segregated. |  |
| 4.2.3.2.10 | All the syringes shall be checked for compliance (of in-process parameter) to the specifications as per the BMR. |  |
| 4.2.3.2.11 | The syringes, which are out of specified limit, shall be labeled as “On Hold” and rest of the syringes shall be cleared if all parameter are within specified limits of BMR. |  |
| 4.2.3.2.12 | Head QA shall take the decision on “On Hold” bottles. |  |
| 4.2.3.3 | Packing Stage : |  |
| 4.2.3.3.1 | Line clearance of the packing area, equipment and process shall be given as per SOP No. QAGE80 (Line Clearance). |  |
| 4.2.3.3.2 | In-process checks shall be performed by IPQA personnel and packing supervisor alternatively at every 1 hour ± 15 minutes during packing activity. |  |
| 4.2.3.3.3 | IPQA personnel and packing supervisor shall randomly check approximately 10 nos. of packed syringes from the running packing line at every 1 hour ± 15 minutes. |  |
| 4.2.3.3.4 | Following in-process checks shall be considered by IPQA personnel and packing supervisor during packing activity:- |  |
|  | - Proper labeling on syringe i.e., availability of label on syringe, proper pasting of label on syringe, product name, generic name, License no. and label claim on label as per BPR, batch coding on label as per BPR (batch no., mfg. date, exp. date, MRP etc.). |  |
|  | - Availability of leaflet (if applicable as per BPR) in the packed carton, printing matter on leaflet, product name, label claim etc. on leaflet. |  |
|  | - Proper pasting of printed box, batch coding on printed box as per BPR (batch no., mfg. date, exp. Date, MRP etc.), product name, generic name, License no. and label claim on printed box as per BPR, printing matter on printed box, colour variation (if any) between two or more printed box, flap of carton. |  |
|  | - Quality of outer box (if applicable as per BPR), label available on outer box, details on label of outer box, numbers of inner box available on outer box as per BPR, proper shrink quality (if shrink applicable). |  |
|  | - Number of containers in the shipper and their packing arrangement. |  |
|  | - BOPP taping quality. |  |
|  | - Coding details on shipper and on shipper label as per BPR. |  |
| 4.2.3.3.5 | Collect the packed control samples and stability samples or any other required samples during packaging operation. |  |
| 4.2.3.3.6 | Verify the yield reconciliation after the completion of packing. |  |
| 5.0 | ANNEXURES:- NA |  |
| 6.0 | REFERENCES:- NA |  |
| 7.0 | REVISION HISTORY:- |  |
| Sr. No.; Revision No.; Date of Revision; SOP No.; Supersedes No.; Reason for Revision. 05; 30/09/2022; QAGE78-05; QAGE78-4; - Change in Firm Name from “Indiana Ophthalmics” to “Indiana Ophthalmics LLP”. - Addition of Annexures, References and Revision History as Section 5.0, 6.0 and 7.0 respectively in the SOP.. |