| 1.0 | OBJECTIVE : | To provide a standardized, procedural guide for the line clearance process at all processing stages to prevent the risk of product mix-ups, cross-contamination and mislabeling. |
| --- | --- | --- |
| 2.0 | SCOPE : | This SOP applies to all line clearance activities in the dispensing, manufacturing, filling, overprinting, and packing areas conducted at Indiana Ophthalmics LLP. |
| 3.0 | RESPONSIBILITY : | Production Designee:- - To initiate the line clearance process and to ensure that the area is clean and free of previous materials. IPQA Person:- - To verify all line clearance activities and to approve the line for the next processing stage. |
| 4.0 | ACCOUNTABILITY : | QA Head |
| 5.0 | PROCEDURE : |  |
| 5.1 | Definition:- |  |
| 5.1.1 | Line Clearance:- |  |
|  | Line clearance is a process which provides a high degree of confidence or assurance that the said line or area is free from any unwanted residue or left over of previous processing’s before proceeding for next process. |  |
| 5.2 | General Guidelines:- |  |
| 5.2.1 | Production designee shall check the area for line clearance. After confirming the area is clean and free of materials, the designee should attach the line clearance label and then ask the IPQA person to approve the line clearance. |  |
| 5.2.2 | Upon receiving a line clearance request from the Production department, IPQA designee shall proceed with the clearance process as outlined in the respective Batch Manufacturing Record (BMR), Batch Packing Record (BPR), or Checklist. |  |
| 5.2.3 | After verifying that every activity for a specific operation is complete, the IPQA designee shall record their observations in the corresponding BMR, BPR, or Checklist. If everything is satisfactory, the IPQA person shall clear the line for the next processing stage by signing the Line Clearance Label in the “Line Clearance Checked by” column. |  |
| 5.2.4 | In the event of any deficiency found during an IPQA check, the IPQA person shall not clear the line for the next processing stage until the deficiency is rectified. All findings shall be reported to the production head for prompt remedial action. |  |
| 5.2.5 | Production head should rectify the deficiencies identified by IPQA person and request again to IPQA personnel for inspection. |  |
| 5.2.6 | IPQA person shall re-inspect the area / machine, etc. for the deficiency noticed originally, if found okay, clear the line for next processing stage. |  |
| 5.2.7 | If the deficiency is not yet rectified, the IPQA person must not clear the line. They must ensure that the next processing stage does not begin without final QA line clearance. This process must be repeated until the deficiency is fully rectified and the line is approved for processing. |  |
| 5.3 | Dispensing Area:- |  |
| 5.3.1 | Line clearance for the dispensing of every new batch shall be taken by store designee and production designee from the IPQA person. |  |
| 5.3.2 | Verify general cleaning of dispensing airlock and dispensing area. There should not be any material spillage on the floor of dispensing airlock and dispensing area. Dispensing airlock and dispensing area shall be free from any previously dispensed materials. Verify the area cleaning log. |  |
| 5.3.3 | Verify that weighing balances available in the area are cleaned as per cleaning procedure of the respective balance. Ensure that daily and monthly calibration of the weighing balance has been performed. Verify daily and monthly calibration record of the respective balance. |  |
| 5.3.4 | Ensure that batch records, dispensing tags and documents of previous product have been removed from the area. |  |
| 5.3.5 | Ensure that filters and pre-filters of RLAF & LAF installed in dispensing area have been cleaned as per pre-determined frequency and verify the record for the same. |  |
| 5.3.6 | Ensure that temperature, humidity and differential pressure of the dispensing area are within the specified limit and same has been recorded in respective format. |  |
| 5.3.7 | Ensure that containers, utensils and dispensing tools which are to be used for the dispensing of next product are cleaned properly. |  |
| 5.3.8 | Ensure that batch record of the product for which materials are to be dispensed is available in the area. |  |
| 5.3.9 | Ensure that line clearance label indicating details of previous product name, previous product batch no., current product name and current product batch no. is attached at the entrance of dispensing room. |  |
| 5.3.10 | All the line clearance activity shall be recorded in the respective batch records. |  |
| 5.4 | Manufacturing Area:- |  |
| 5.4.1 | Check general cleanliness of the manufacturing area & airlock. |  |
| 5.4.2 | Ensure that batch records, labels, polybags, vessels, hand gloves or any other accessories used in the manufacturing of previous product are removed from the area. |  |
| 5.4.3 | Ensure that tools, accessories and utensils such as spatula, scoops, trolley etc. are cleaned properly. |  |
| 5.4.4 | Ensure that manufacturing vessels and storage vessels are cleaned properly and cleaning status is available on it. |  |
| 5.4.5 | Ensure availability of BMR of the product to be manufactured and ensure that BMR is complete up to the respective stage. |  |
| 5.4.6 | Ensure that temperature, humidity and differential pressure of the manufacturing area is within the limit. Record the temperature, humidity and differential pressure of the manufacturing area in line clearance checklist. |  |
| 5.4.7 | Ensure that all daily records such as cleaning records, personnel hygiene records and all other relevant records are filled completely. |  |
| 5.4.8 | Concern production designee and IPQA person shall verify all the details as mentioned in Annexure-I (Line Clearance Checklist for Manufacturing) and shall sign in respective columns. |  |
| 5.4.9 | After satisfactory verification of all the checkpoint as per Annexure-I, IPQA person shall sign in the respective Line Clearance Label in the column of “Line Clearance Checked by” as per Annexure-V. |  |
| 5.5 | Filtration Process:- |  |
| 5.5.1 | Check general cleanliness of the filtration area. |  |
| 5.5.2 | Ensure that area is free from any type of unwanted items and utensil. |  |
| 5.5.3 | Ensure the cleaning of membrane filtration unit and verify the filters which are to be used for filtration process. |  |
| 5.6 | Filling Area:- |  |
| 5.6.1 | Check general cleanliness of the filling area and equipments. |  |
| 5.6.2 | Ensure that the area and filling line is free from any previous product. |  |
| 5.6.3 | Ensure that batch records, labels, vessels, primary packing materials, or any other accessories used in the filling of previous product are removed from the area. |  |
| 5.6.4 | Ensure that filling machine and accessories are cleaned and cleaning status is available on it. |  |
| 5.6.5 | Ensure that temperature, humidity and differential pressure of the filling area is within the limit. Record the temperature, humidity and differential pressure of the filling area in line clearance checklist. |  |
| 5.6.6 | Ensure that LAF is in ON condition. Record the differential pressure of LAF (Limit:-7 to 20 mmwc) in the checklist. |  |
| 5.6.7 | Ensure that batch record of the product to be filled is available and complete up to the respective processing stage. |  |
| 5.6.8 | Ensure that bulk product is released by QC department and bulk COA is available with respective BMR. |  |
| 5.6.9 | Ensure that all daily records such as cleaning records, personnel hygiene records and all other relevant records are filled completely. |  |
| 5.6.10 | Concern production designee and IPQA person shall verify all the details as mentioned in Annexure-II (Line Clearance Checklist for Filling) and shall sign in respective columns. |  |
| 5.6.11 | IPQA person shall sign in the respective Line Clearance Label in the column of “Line Clearance Checked by” as per Annexure-V. |  |
| 5.7 | Sterilization Area:- |  |
| 5.7.1 | Check general cleanliness of sterilization area. |  |
| 5.7.2 | Ensure that autoclave and sterilization area is free from any previous product. |  |
| 5.8 | Overprinting Area:- |  |
| 5.8.1 | Check the general cleanliness of the overprinting area. |  |
| 5.8.2 | Ensure that overprinting machine and area is free from any type of previous product. |  |
| 5.8.3 | Ensure that cartons, labels, stereo, batch records etc. of the previous product is removed from the area. |  |
| 5.8.4 | Ensure that overprinting machine is cleaned and cleaning status is available on it. |  |
| 5.8.5 | Ensure that previously used stereos are destroyed. |  |
| 5.8.6 | Ensure that batch record of the product carton to be overprinted is available and complete up to the respective processing stage. |  |
| 5.8.7 | Ensure that overprinting machine logbook is available and online. |  |
| 5.8.8 | Ensure that issued cartons are released by QC and ensure that Approved labels are available on the containers. |  |
| 5.8.9 | Ensure that stereo of the product carton to be overprinted is available and impression of the same is taken in the stereo register. |  |
| 5.8.10 | Verify AR No. against packing material dispensing slip. |  |
| 5.8.11 | Concerned packing supervisor and IPQA person shall verify all the details as mentioned in Annexure-III (Line Clearance Checklist for Overprinting) and shall sign in respective columns. |  |
| 5.8.12 | IPQA person shall sign in the respective Line Clearance Label in the column of “Line Clearance Checked by” as per Annexure-V. |  |
| 5.9 | Packing Area:- |  |
| 5.9.1 | Prior to start of packing activity of respective batch, the packing line shall be checked for absence of previous product’s materials / containers / vials / rejections etc. by Packing Supervisor / Designee and IPQA person shall verify it. |  |
| 5.9.2 | Ensure that bottles / tubes / syringes, cartons, boxes / wrapper or any other packaging material of the previous product are completely removed from above and below the packing table. |  |
| 5.9.3 | Ensure that packing table and conveyor belt are properly cleaned. |  |
| 5.9.4 | Ensure that dustbin is completely empty. |  |
| 5.9.5 | Ensure that details of the previous product are erased from status board & details of the product to be packed are filled on status board. |  |
| 5.9.6 | Ensure that weighing balance to be used for weighing of tubes / shrink bags / outer boxes are free from any remaining’s of previous product & are properly cleaned. |  |
| 5.9.7 | Ensure that rejected bottles / tubes / syringes, cartons, leaflets, labels etc. of the previous product / batch no. are sent for destruction. |  |
| 5.9.8 | Ensure that Packed shippers of previous product are shifted to BSR. |  |
| 5.9.9 | Ensure that Batch Packing Record of the previous product is completed & submitted to QA dept. for review. |  |
| 5.9.10 | Check that correct packing material issued with respect to item code, name of materials, A.R. No., Required qty. of PM, Issued qty. of PM as mentioned in BPR. |  |
| 5.9.11 | Check the approved label details on the issued packing materials. |  |
| 5.9.12 | Check the overprinting details on the packing materials. |  |
| 5.9.13 | Transfer the bottles / tubes / syringes crates of the batch to be packed into packing area & check the detail on the label of each crate. |  |
| 5.9.14 | Ensure that sealed crate for rejected packing material is available for next batch to be packed. |  |
| 5.9.15 | If any one of the above mentioned points doesn’t comply, IPQA person shall not give line clearance for packing line. |  |
| 5.9.16 | Concerned packing supervisor and IPQA person shall verify all the details as mentioned in Annexure-IV (Line Clearance Checklist for Packing) and shall sign in respective columns. |  |
| 5.9.17 | IPQA person shall sign in the respective Line Clearance Label in the column of “Line Clearance Checked by” as per Annexure-V. |  |
| 6.0 | ANNEXURES:- |  |
| 6.1 | Annexure-I : Line Clearance Checklist for Manufacturing |  |
| 6.2 | Annexure-II : Line Clearance Checklist for Filling |  |
| 6.3 | Annexure-III : Line Clearance Checklist for Overprinting |  |
| 6.4 | Annexure-IV : Line Clearance Checklist for Packing |  |
| 6.5 | Annexure-V : Format for Line Clearance Label |  |
| 7.0 | REFERENCES:- NA |  |
| 8.0 | REVISION HISTORY:- |  |
| Sr. No.; Revision No.; Date of Revision; SOP No.; Supersedes No.; Reason for Revision. 06; 27/11/2022; QAGE80-06; QAGE80-5; - Change in Firm Name from “Indiana Ophthalmics” to “Indiana Ophthalmics LLP”. - Addition of Annexures, References and Revision History as Section 6.0, 7.0 and 8.0 respectively in the SOP. - Merging of SOP No. QAGE27 (Line Clearance in Dispensing Area), SOP No. PRPA10 (Line Clearance for Packing) and SOP No. QAGE80 (Line Clearance) in single SOP No. QAGE80 as Line Clearance.. 07; 14/08/2024; QAGE80-07; QAGE80-06; - Addition of Scope as section 2.0 in the SOP. - Step 5.6.6 incorporated as “Ensure that LAF is in ON condition. Record the differential pressure of LAF (Limit:-7 to 20 mmwc) in the checklist”. - Annexure-I updated by adding provision for recording the temperature, humidity and differential pressure in manufacturing area during line clearance. - Annexure-I and Annexure-II updated by adding checkpoint as “Ensure that LAF is in ON condition. Record the differential pressure of LAF (Limit:-7 to 20 mmwc)”.. 08; 12/10/2025; QAGE80-08; QAGE80-07; - Annexure-I, Annexure-II, Annexure-III and Annexure-IV updated by adding provision for recording date and time during line clearance.. 09; 12/12/2025; QAGE80-09; QAGE80-08; - Addition of Start Time and End Time in Annexure-I, Annexure-II, Annexure-III and Annexure-IV.. |